PDU

PDU won’t turn on

Verify PDU power cord (yellow) is securely fastened at each end

Verify wall outlet/strip is good

Check F1 on rear of PDU

Verify the PDU and Deck Recorder are plugged into separate 110VAC circuits

PDU turns on but small confidence monitors aren’t on

Verify power strip on rear of PDU is turned on (physical switch in rear)

480 VAC cuts out immediately after turning it on

Rare occurrence due to the high surge current of 2 transformers in series which can happen at AC zero crossings – try again.

Ground fault alarm is activated

Check the Lander humidity reading on the PDU display

Nominally, should read 30% or less.

If above 50%, could indicate water intrusion into the Lander.

Turn off 480VAC, press GF reset, and attempt to power on 480VAC again.

Power down entire system and attempt to power on again.

If GF persists, pull lander from ocean (if deployed) and attempt on deck (dry)

Inspect tether for any damage/kinks

Touch panel is unresponsive

Reboot the entire PDU

No telemetry or video or cameras will not power on (no camera feedback to display)

Be sure to wait ~30 seconds after applying 480VAC due to Lander PLC boot up time

Verify all cables to the lander are connected.

Verify the 480VAC is actually turned on

Most likely due to a poor fiber connection

Clean the fiber and bulkhead connector on Lander endcap

Clean the fiber and bulkhead connector on Lander J-Box

If cleaning the fiber ends doesn’t fix the problem, inspect the tether for physical damage (see Tether section)

Deck Recorder

Deck Recorder won’t turn on

Verify the deck recorder is plugged into a good outlet/strip

Verify the PDU and Deck Recorder are plugged into separate 110VAC circuits

HTML page is unresponsive

Reboot the entire Deck Recorder

Lander

Light(s) not turning on

Verify the PDU is on and the 480VAC is active

Verify that the PDU is getting telemetry and there is video being displayed

Verify the variable dimming value is not set to zero

Verify the 6-pin cables for each light is plugged into the correct connector on the endcap

Tether

Loss of video/telemetry (if steps from PDU section do not fix the problem)

Inspect the tether for physical damage (such as major twists or a slice or nick to the jacket)

This may require lowering the lander into the water to fully inspect the length of tether which was used when the problem was noticed.

Light transmission verification

Disconnect the one meter fiber patch cord from the lander’s junction box and attach a visible light source (from a laser test set) to the ST connector.

Open the outer junction box on the winch and disconnect the ST fiber.

Turn on the visible light source; you should see light at the winch fiber end.