- Features

  - Quad, Picture in Picture, Full Screen, and Custom display modes (Custom Display Mode: the size, position and selection of any or all windows are customizable)

  - 2- and 3-source viewing is possible with this method

  - Maintain input aspect ratio if desired-no stretching necessary

  - Layer display windows and adjust transparency of each video source display

  - Independent video in to video out resolution

  - Supports 2K resolution 2048x1080, HDTV resolutions to 1080p, and computer resolutions to 1920x1200

  - HDMI features supported:

  - Inputs: 24-, 30-, and 36-bit xvYCC, sRGB, and YCbCr

  - Outputs: 24- and 30-bit sRGB

  - Four-channel non-mixing or one channel mixing stereo with 16-, 20-, or 24-bit uncompressed PCM audio

  - Bandwidth up to 165 MHz

  - Inputs: 2.25 Gbps

  - Output: 2.0625 Gbps

  - HDCP compliant

  - Fluid, real-time video performance with 60 frames per second (fps) in all four quadrants

  - Zoom, pan, and crop the image from any source to focus on key areas

  - Switch audio independently of video from connected HDMI sources (e.g. source 1 and source 2 video are active while only source 3 audio is active)

  - Multiple audio sources can be simultaneously active in any display mode or preset layout

  - Adjustable audio gain with VU level indicators for each input channel

  - A customized text label can be added for each video window to provide easy input identification (UMD)

  - Any DVI source or display can be connected by using the DVI-HD-xx-MM cable (not included)

  - Use DVIA-HD-CNVTR-LC or DVI-HD-CNVTR DVI + Audio to HDMI Converters to pass and independently switch audio signals to the multiviewer

  - Control the multiviewer through Ethernet, RS232 serial port, on screen display (OSD), front panel buttons, or IR remote

  - Cascade multiviewers to display video from any number of video sources on one screen

  - Each video source in the cascade can be controlled directly from the Web Server graphical user interface under Custom Mode Settings

  - Backup and restore multiviewer configurations

  - Supported output resolutions can be selected or set to auto detect optimal resolution from the monitor's EDID

  - Desktop unit

- Specifications

  - Video Input

  - Four female HDMI connectors.

  - Supported video input resolutions:

  - 2K Cinema (2048x1080 @ 60Hz).

  - HDTV resolutions to 1080p.

  - Computer resolutions to 1920x1200.

  - Supports digital HDMI devices, such as DVD/Blu-ray players, satellite receivers and HDTV tuners.

  - Multiplatform support: Windows 2000/XP/Vista/7/8, Windows Server 2000/2003/2008, Solaris, Linux, FreeBSD, and MAC OS 9/10.

  - A DVI source can be connected by using the DVI-HD-xx-MM cable (not included). The cable does not pass audio to the multiviewer.

  - Video Output

  - One female HDMI connector.

  - Supported video output resolutions:

  - 2K Cinema (2048x1080 @ 60Hz).

  - HDTV resolutions to 1080p.

  - Computer resolutions to 1920x1200.

  - HDMI-embedded audio switching (four-channel stereo, non-mixing or one-channel stereo, mixing).

  - A DVI display can be connected by using the DVI-HD-xx-MM cable (not included). The cable does not pass audio to the display.

  - Protocols

  - HTTP, HTTPS.

  - TCP/IP, DHCP, UDP, ARP.

  - IPV4.

  - Power

  - 110 to 220 VAC at 50 or 60Hz via country-specific AC adapter.

  - Power consumption: 10W

  - Environmental

  - Operating temperature: 32 to 122°F (0 to 50°C).

  - Storage temperature: -22 to 140°F (-30 to 60°C).

  - Operating and storage relative humidity: 17 to 90% non-condensing RH.

  - Dimensions WxDxH (in): 7.35x4.98x1.09 (187x126x28 mm).

  - Regulatory Approval: CE, RoHS, TAA compliant.

- Item Includes

  - 1x SPLITMUX-HD-4RT multiviewer

  - 1x country-specific power cord

  - 1x DB9 female to RJ45 female adapter

  - 1x 5-ft CAT5e super flat patch cable

  - 1x IR remote control with two AAA batteries

  - 1x CD that contains the manual and NTI Discovery Tool