Sony Multi / Micro USB Terminal

M1: Vbus (+5V, 2A max)

M2: USBD-

M3: USBD+

M4: USB_ID

M5: GND

M6: +3.1V (for LANC_DC)

M7: reserved

M8: LANC_SIG

M9: reserved

M10: SELECT (resistor to M6 or M14)

M11: Audio R Out / Activate Camera / Focus

M12: Audio L Out / Release Shutter

M13: Composite Video Out

M14: GND

M15: reserved

| U1 | VBUS (+5V, max 2A) |
| --- | --- |
| U2 | USB D- |
| U3 | USB D+ |
| U4 | USB ID |
| U5 | GND_USB |
| — |  |
| M10 | +3.3V (MULTI_DC) |
| M09 | reserved |
| M08 | “UART_TX” / “LANC_SIG” (At A6000 it seems to be UART; No LANC!) |
| M07 | “UART_RX” / “BOOT_IN” (At A6000 it seems to be UART; No Bootloader/Debug) |
| M06 | SELECT (resistor against M01 or M09 for selecting Multiport functionalitites/protocol) |
| M05 | Short to ground to focus, Audio R out |
| M04 | Short to ground to take image, Audio L out |
| M03 | Maybe composite video out (no AV-out at A6000 known to me!) |
| M02 | GND (identical to GND_USB) |
| M01 | Power_On/Off (short to GND) |

- 1: "VBUS" - 5V-Eingangsspannung vom USB-Host

- 2: "D-" - USB-Datenleitung (negativ)

- 3: "D+" - USB-Datenleitung (positiv)

- 4: "USB_ID" - für USB-OnTheGo

- 5: "GND" - in der Kamera mit Pin 14 ("AV_GND") verbunden

- 6: "D_3.1V" / "LANC_DC" - Ausgang für 3.1 bis 3.3V Spannung

- 7: "XRESET_REQ" - Eingang für Reset-Anfrage

- 8: "UART_TX" / "LANC_SIG" - optionaler Ausgang für serielle Schnittstelle (Debug) oder LANC

- 9: "UART_RX" / "BOOT_IN" - optionaler Eingang für serielle Schnittstelle (Debug) und Bootloader

- 10: "AD_JACK_IN" - Analoger Eingang für Widerstand zur Erkennung des angeschlossenen Kabel-/Zubehörtyps

- 11: "LINEOUT_R" / "XAE_LOCK_SW" - optionaler Audioausgang (rechts, in der Kamera über Kondensator entkoppelt) und/oder Eingang für Auslöser (1. Stufe) im Fotomodus der Kamera (und mit richtigem Widerstand an AD_JACK_IN)

- 12: "LINEOUT_L" / "XSHUT_SW" - optionaler Audioausgang (links, in der Kamera über Kondensator entkoppelt) und/oder Eingang für Auslöser (2. Stufe) im Fotomodus der Kamera (und mit richtigem Widerstand an AD_JACK_IN)

- 13: "VIDEO" - optionaler Composite-Video-Ausgang

- 14: "AV_GND" - in der Kamera mit Pin 5 ("GND") verbunden

- 15: "XPWR_ON" - Eingang für externen ON/OFF-Taster (gegen GND)

Figure 1: Multiport connector pin numbers

Figure 2: Multiport connector pin names

Pinout according to Figures 1 and 2:

USB

A1  Vbus (+5V, 2A Max)

A2  USB Data -

A3  USB Data +

A4  USB ID

A5  Gnd

Multiport

1  +3.1V (LANC DC)

2  Reserved

3  UART Tx

4  UART Rx

5  Select (Resistor to pin 1 or 9)

6  Audio Out R / Focus

7  Audio Out L / Shutter

8  Composite Video Out

9  Gnd (same as USB Gnd)

10 Camera Power On/Off

Figure 3: Sony Multiport 10 pin signals

Figure 4: Sony Multiport USB signals