Benthic Observation Survey System (BOSS) Functional Requirements

Must be able to be deployed from ships of opportunity

Must be a tethered system

Must have a load bearing tether

Must be able to launch and recovery quickly

Must be able to operate in sea state two

Must be able to land on rocky habitats with as little impact as possible

Must be able to stay on bottom for five minutes

Must be able to hopscotch (be towed) between sites during each dive

Must have swappable bases for multiple sea floor types

Must be able to record stereoscopic video in (near) 360 degrees

Must have the ability to remotely control iris/ISO of the cameras

Must provide adequate lighting for all cameras

Must have the ability to add in contextual sensors

Must be able to attach transponder onto lander

Must have a downward looking camera to assist with positioning

Must provide downward video and lander depth to bridge and winch operator

Must be able to provide live video topside

Must be able to record topside

Must provide synchronization between the multiple cameras

Must combine meta-data (telemetry) into one file for each recording session