Housing Options for Benthic Observation System

1. Single Housing

Has to be large enough to accommodate all of the cameras and electronics

Cameras: 2 to a side, 3 sides, spacing between cameras TBD

Minimizes number of external connectors and cablings (less $ and failure points)

Will be large and bulky (and buoyant)

Will allow switching between real time view between 4 cameras.

External connector(s) and cables to lights (# TBD)

External connector(s) and cables to downward looking camera and light

2. Main Housing with 3 Camera Housings (2 cams/housing)

Inter housing connectors and cabling required (6 connectors & 3 cables)

More deployment flexibility

If real time view is needed, 6 additional large connectors & 3 high speed cables required

Main housing will have external connector(s) and cables to lights (# TBD)

Main housing will have external connector(s) and cables to downward looking camera and light

3. Main Housing with 6 Camera Housings (1 cam/housing)

Inter housing connectors and cabling required (12 connectors & 6 cables)

Maximum design flexibility

If real time view is needed, 6 additional large connectors & 3 high speed cables required

Main housing will have external connector(s) and cables to lights (# TBD)

Main housing will have external external connector(s) and cables to downward looking camera and light