ECO-2000 Winch

Factory Acceptance Test Plan

Model Number: ECO-03-075-L394SLB-CUST-PD-CR

Client Name: MBARI

Job Number: 50021

Serial Number: 485-001

| Document Number: | ECO-2000-PLN-485-001 |
| --- | --- |
| Document Name: | ECO Winch Factory Acceptance Test Plan |
| Document Revision: | REVISION 1.0 |
| OST Point of Contact: | Don Brockett Office: 425-869-1834 Fax: 425-869-5554 |

| OST DOCUMENT No.: ECO-2000-PLN-485-001 |  |  |  |
| --- | --- | --- | --- |
| Release No.: | 1.0 | Checked | dd-mmm-yy |
| Prepared By: | Don Brockett dbrocket@okeanus.com COO |  | Date: 8/31/18 |
| Released By: | Ryan Marsh rmarsh@okeanus.com Mechanical Engineer |  | Date: 8/31/18 |

The information contained in this document is the proprietary property of Okeanus Science and Technology, LLC. External use of this document shall be limited to the purposes of information and evaluation only. Any commercial use for manufacturing is strictly prohibited. Release of this document or any information contained herein to third parties is prohibited without written permission from OST.

EXECUTIVE SUMMARY

This document outlines the plan for performing functional testing of OST’s standard ECO-2000 Winch, and serves as the Factory Acceptance Test (FAT) document for winches undergoing the FAT.

TABLE OF CONTENTS

1	Introduction	5

1.1	Scope	5

1.2	Purpose	5

1.3	References	5

1.4	Regulations	5

1.5	Abbreviations and Acronyms	5

1.6	Classification and Distribution Rights	5

2	ECO Winch Test Objectives	6

2.1	Visual Inspections	6

2.2	No Load Speed Test	6

2.3	Load Test	6

2.3.1	Brake Test	6

2.3.2	Dynamic Load Test	6

3	Organizational Responsibility and Test Management	6

4	Tests and Examinations	7

4.1	Sequencing of Tests	7

4.2	Test Requirements	7

4.2.1	Mechanical	7

4.2.2	Tools and Equipment	7

4.3	Test Program	8

4.3.1	Visual Inspection	8

4.3.2	Functional Testing	9

- Introduction

## Scope

ECO Winch Test Plan

The ECO-2000 Winch will be tested to ensure it is fully functional and meets design specifications as outlined in the following paragraphs.

## Purpose

Test and Evaluation Program Overview

The ECO Winch Test Plan describes parameters OST will test and describes how they will be tested. The plan is comprised of visual, load and speed components described in detail in this document. The testing outlined in this document does not comply with any regulatory requirements but uses the “American Bureau of Shipping | Rules for Building and Classing Underwater Vehicles, Systems and Hyperbaric Facilities, 2015, Section 17” as a guideline for this effort.

The OST plan outlines the broad objectives of this test and the order in which they are performed.

## References

American Bureau of Shipping | Rules for Building and Classing Underwater Vehicles, Systems and Hyperbaric Facilities, 2015, Section 17.

## Regulations

The ECO Winch has been manufactured using American Bureau of Shipping standards as a general guideline.

## Abbreviations and Acronyms

m/s: Meters Per Second

fpm: Feet Per Minute

rpm: Revolutions Per Minute

SOSI: Sound Ocean Systems, Inc.

## Classification and Distribution Rights

Approved for release to Customer and/or Customer approved recipient and to OST document control.

- ECO Winch Test Objectives

This ECO Winch Factory Acceptance Test Plan outlines the steps necessary to verify that the ECO Winch meets or exceeds the following performance specifications:

- Line Speed 			121 ft./min (0.61 m/s) Full drum

98 ft./min (0.50 m/s) Bare Drum

- Line Pull			600 lbs. @ Full drum

750 lbs. @ Bare Drum

- Brake Holding Capacity	1,050 lbs. Full Drum

1,312 lbs. Bare Drum

The test program will be broken into four (4) distinct sections including visual inspections (unpowered), no load speed tests, static load tests, and dynamic load tests. In each of these phases the system will be inspected against the check lists in the following sections.

All of the system functionality will be validated during these tests.

## Visual Inspections

The sub-systems on the winch will be inspected in detail for quality workmanship and correct installation.

## No Load Speed Test and Payout Testing

The maximum bare drum line speed of the winch in both the pay-out and haul-in directions under no-load conditions will be verified during this test.  The speed and payout display (if equipped) will also be tested at this time.

## Load Test

### Brake Test

The ECO brake test will be configured with a load of approximately 1,312 lbs. at bare drum for the static load test. This load will be maintained on the winch for a total of 1 minute.

### Dynamic Load Test

Subsequent to the static load test the load will be adjusted down to the rated Safe Working Load of the winch, 750 lbs. The winch will be operated under this load in both the haul-in and the pay-out directions in order to confirm the winch’s ability to properly function under maximum rated load.  In both operational scenarios, winch in/out, the joystick will be released to its neutral position and the load should ramp down to a stop.

## Additional Functional and Configuration Testing

This includes other miscellaneous tests and inspections of the equipment not mentioned above.  Examples include E-stop functionality, enclosure water ingress test, documentation of cable spooled onto the winch, and other tests required for this specific winch.

- Organizational Responsibility and Test Management

The factory acceptance test will be conducted under the direction of the Test Director who will distribute test results to the customer via email upon request.

- Tests and Examinations

## Sequencing of Tests

The schedule of testing and evaluation for the FAT is scheduled as follows:

- Visual Inspection Parameters

- Functional testing

## Test Requirements

To perform the tests outlined in this document the following equipment and preparations will be required.

### Mechanical

The main winch structure will be properly secured to a concrete deck or other suitable foundation.

### Tools and Equipment

NOTE:

All lifting equipment will have proper load certificates and all load cells/measurement devices will have current calibration certificates

- Test rope provided by Okeanus

- General Tool Kit

- Digital Volt Meter

- Load cell -5,000 lb.

- Forklift/Lift

## Test Program

The Test Program is an Okeanus procedure designed to ensure that the specifications are satisfied and the functional capability of the winch is documented.  This testing program will be performed in Redmond, Washington.

### Visual Inspection Parameters

The following parameters will be visually inspected during operation of the ECO Winch.  Note that in some cases, purchase orders and certificates of conformance may be used to demonstrate conformance.

| Parameter | Results | Signature |
| --- | --- | --- |
| All aluminum components are powder coated with a marine grade powder coat where appropriate |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| Dissimilar metals have been isolated where appropriate |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| All fasteners are 316 stainless steel where appropriate |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| The winch is marked with the manufacturer’s name or symbol, model number, serial number, and manufacturing date |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| There are no cracks breaks, chips, bends, burrs, loose attaching parts, loose electrical connections or any evidence of poor workmanship for the purpose intended |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| The unit is clean and free of foreign material at time of shipment |  |  |
| Meets requirement | Pass/Fail |  |

| Parameter | Results | Signature |
| --- | --- | --- |
| The ground wire from the enclosure to the winch frame has been installed |  |  |
| Meets requirement | Pass/Fail |  |

### Functional Testing

The functional testing will be performed with the ECO Winch as follows:

- Operate the winch in and out at maximum speed under no load.  Allow the winch to ramp up to its maximum speed before taking the measurement. If the drum speed meets or exceeds 26.5 rpm, the winch passes.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| RPM / Speed | Haul-In |  |  |
| RPM / Speed | Pay-Out |  |  |
| Meets Requirement | 26.5 rpm | Pass/Fail |  |

- Verify Counter Payout Length Accuracy.  Zero the cable counter with the button on the controller and note the position of the free end of the cable.  With enough tension on the cable to ensure no slippage over the sheave occurs; pay-out the cable a fixed distance on the counter, such as 50 feet.  Stop the winch.  Measure the length of the cable that has been paid out.  The measured length of cable should be within +/- 5% of the distance indicated by the counter in order to pass. Haul-in the cable under slight tension until the free end of the cable returns to its start position.  Record the counter reading.  The distance should be within +/- 5% of the length hauled in to pass.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| Actual Reading | Measured Cable |  |  |
| Counter Reading | Pay-Out Reading |  |  |
| Payout Results | % Error |  |  |
| Actual Reading | Measured Cable |  |  |
| Counter Reading | Haul-In Reading |  |  |
| Haul-In Results | % Error |  |  |
| Meets requirement | +/- 5% | Pass/Fail |  |

- Verify Counter Payout Rate Accuracy.  This test assumes that the above test is a pass and only verifies that the rate displayed matches the payout measured. Zero the cable counter with the button on the controller.  Payout at a constant speed (as per shown by the display) for approximately 1-3 meters (as shown on the display) while timing with a stop watch.  Calculate the average rate and compare to what was displayed on meter during the test.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| Haul-In Speed on display | m./min. |  |  |
| Meters hauled in from display | M |  |  |
| Time | (s) |  |  |
| Measured rate | m./min. |  |  |
| Meets requirement | +/- 5% | Pass/Fail |  |

- Verify Lifting Capacity. Load test cable with 750 lbs. line tension. Rig the winch cable for lifting a suspended 750 lb. load bare drum.  Haul-in and pay-out about 15 feet of cable under load. If the winch is able to haul-in and pay-out the load smoothly and under control, the winch passes.  A vertical fleet angle of approximately 15° to 80° should be used during testing.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| Load | Measured Load |  |  |
| Line Pull – Full Drum | Pay In |  |  |
| Load | Measured Load |  |  |
| Line Pull – Full Drum | Pay Out |  |  |
| Meets requirement | 750lb. load | Pass/Fail |  |

- Verify Brake Capacity.  Perform static load test at a minimum of 1,312 lbs. at bare drum. Rig the winch cable for lifting a suspended load.  With the winch joystick in the neutral position attach a 1,312 lbs. load to the cable.  This can be done by come-along, forklift, etc.  If the cable does not payout, the winch passes.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| Static Load Test | Brake / Load lbs. |  |  |
| Time Under Load | Time / min |  |  |
| Meets Requirement | 1,312 lbs. | Pass/Fail |  |

- Levelwind Operation. Verify the levelwind traverses the full width of the drum.  While maintaining slight tension on the cable, pay-out sufficient cable to cause the levelwind carriage to traverse across the full width of the drum and head back in the opposite direction.  Note that traversing the full width of the drum will require reversing direction twice, once at each end.   Stop the winch and repeat the process in the haul-in direction.  (‘Proper Winding’ is used in this document to describe the expected winding or wrapping of the cable onto the drum. Ideally, all wraps are identical and equally spaced, and each wrap of an additional layer is a mirror-image copy of the wrap of the previous layer. In practice, imperfections are expected. A ‘Proper Winding’ cable is one that results with the entire cable wrapped on the drum and subsequent deployment does not damage the cable. Variations in wrap spacing are acceptable.)

If the carriage travels the full length of the drum with no binding, excessive gaps in cable lay, or overlays, the winch passes.

| Parameter | Function | Results | Signature |
| --- | --- | --- | --- |
| Level Wind Operation | Winch in/out |  |  |
| Meets requirements | Traverses Length of Drum | Pass/Fail |  |

- Verify that braking resistor thermal protection switch is functional

| Parameter | Results | Signature |
| --- | --- | --- |
| Disconnect the thermal switch to simulate a resistor over-temp event and verify that the payout function is inhibited. Also verify that Haul-in function remains operational. |  |  |
| Meets Requirements | Pass/Fail |  |

- Verify that E-Stop if functional

| Parameter | Results | Signature |
| --- | --- | --- |
| Depress E-stop and verify that the winch can neither payout or haul in. |  |  |
| Meets Requirements | Pass/Fail |  |

- Verify that the electrical enclosure is watertight

| Parameter | Results | Signature |
| --- | --- | --- |
| Spray the electrical enclosure with a garden hose from all angles and directions for approximately 5 minutes. |  |  |
| Meets Requirements | Pass/Fail |  |

- Verify remote control operation.  Switch control location from Local to Remote and perform the following tests.

| Parameter | Results | Signature |
| --- | --- | --- |
| Joystick | Use the remote control joystick to pay out and haul in on the winch. If the winch does so under correct direction, the test results in a pass. |  |
| E-Stop | Depress the E-Stop switch, the use the remote control joystick to attempt to operate the winch. Then use the local control joystick to attempt to operate the winch. If the winch does not turn during either joystick attempt, the test results in a pass. |  |
| Display | Confirm the line speed displayed on the remote matches that on the local control station. Confirm the line scope displayed on the remote matches that on the local control station. |  |
| Local Control | When in remote control location mode, attempt to operate the winch with the local controller. If the winch does not respond, the test results in a pass. |  |

- With the 750 lb safe working load suspended from a sheave, raise and lower the load at low speed and maximum speed and record VFD current to the motor.

| Speed | Haul-In Current (A) | Pay-Out Current (A) | Signature |
| --- | --- | --- | --- |
| Minimum Speed |  |  |  |
| Maximum Speed |  |  |  |

- Final Acceptance

If the unit has passed all testing, sign document below. List any conditions of final acceptance, if necessary.

| Authorized By: | Signature | Date |
| --- | --- | --- |
| Witnessed By Project Manager: |  |  |
| Test Performed By: |  |  |
| QC Acceptance: |  |  |