nsFOCE Chamber (Electronics) Housing Build

Drawings Referenced

System Level:

nsFOCE Cabling Layout Diagram MpHOx.pdf

nsFOCE Single Chamber Block Diagram.pdf

Chamber Housing:

nsFOCE Chamber Housing Block Diagram MpHOx.pdf

nsFOCE Chamber Housing Wiring Diagram MpHOx 1of2.pdf

nsFOCE Chamber Housing Wiring Diagram MpHOx 2of2.pdf

Cables:

W07 – Chamber to Valve Connection Oil Filled Whip.pdf

W08 – Valve Connection to Valve Housing Oil Filled Whip.pdf

W09 – Chamber to Durafet Connection Oil Filled Whip.pdf

W11 – Chamber to Ambient Durafet Connection Oil Filled Whip.pdf

Notes

Per nsFOCE Cabling Layout Diagram MpHOx.pdf, there are two configurations of the Chamber Housings. A and D will have two additional output whips (W11A and W11D) to feed a group of ambient sensors. A is going to connect to the sensors, while D will have these whips dummied off for future use. B and C will have these connections capped off at the endcaps.

The parts (internal electronics + cables/whips) for chamber D are the operational parts from the 2018 proto deployment. They’re in an oil filled housing currently; Mike Parker can help drain it.

Pre-Build Oil-Filled Whips

W7 (x4)

W8 (x4)

W9 (x8)

W11 (x4)

Build each cable per specified length plus an additional 1-2 feet to account for wire lengths inside of the Chamber Housing.

Dale had to re-order the blue tubing for the oil filled whips. It should be here by the time this work is going on.

Note the different colors of locking sleeves (noted on individual drawings)