nsFOCE Spares

Micro Node

This is not an easily serviced module due to being permanently connected to the subsea tether.

Potential failure points:

Yellowbrik EO Module PN# OBD 1510E

Vicor DC-DC (Micro) Converter (12Vdc, 75W) PN# V375C12C75BN [$200]

Seaconn Bulkhead Connector PN# MINK16FCR [$500]

Distribution Node

This is a diver serviceable module, but removing it disables the entire nsFOCE experiment.

Potential failure points:

Vicor DC-DC (Micro) Converter (24Vdc, 150W) PN# V375C24C150BN [$200]

Sensata (Crydom) Solid State Relay PN# D5D07 [$120]

Wago Stack

Coupler PN# 750-352 [$450]

Power Supply PN# 750-610 [$50]

Octal Digital Output PN# 750-530 [$150]

Chamber Node(s)

There are four chamber nodes, one per flume. These are diver serviceable. One or more can be removed and keep nsFOCE operating, albeit, at reduced functionality. The thruster module is the only external housing that is permanently tethered to the chamber node.

Potential failure points:

Vicor DC-DC (Micro) Converter (24Vdc, 150W) PN# V375C24C150BN [$200]

Vicor DC-DC (Micro) Converter (12Vdc, 75W) PN# V375C12C75BN [$200]

Vicor DC-DC (Micro) Converter (12Vdc, 150W) PN# V375C12C75BN [$200]

Digi Quad Serial Server (x2) PN# 70002045 [$550]

Wago Stack

Coupler PN# 750-352 [$450]

Power Supply PN# 750-602 [$50]

Dual Digital Output PN# 750-502 [$120]

Dual Analog Input (4-20ma) PN# 750-454 [$400]

Dual Analog Input (RTD) PN# 750-461 [$450]

Dual Analog Output PN# 750-550 [$400]

Octal Digital Output PN# 750-530 [$150]

Valve Module

There are four valve modules and they are diver serviceable.

Potential failure points

Hanbay Actuator PN# MCM-000AF-3-SS-18RS8 [$2300]

Thruster Module

There are four thruster modules; they are permanently attached to the chamber module via a Falmat cable with gland fittings on both ends. Both must be recovered simultaneously.

Potential failure points:

Elmo Whistle Motor Controller PN# G-SOLTWIR50/100ER1 [$1100]

Vicor DC-DC (Maxi) Converter (48Vdc, 600W) PN# V375A48C600BN [$450]

Durafet Module

There are ten Durafet modules and they are diver serviceable. Removing it causes the loss of control of the Seabird pump, Durafet pH probe, Aanderaa optode, and Aanderaa conductivity sensors.

Potential failure points:

MpHOx board PN# MBARI custom, rev A

MCAP board PN# MBARI custom, rev D

Bias batteries PN# Double AA’s (x2) [note: sized for one year of continuous use]

Honeywell Durafet Sensor PN#

System Level

Potential failure points:

Subconn cables (various pin counts) PN#

Maintenance

Replacement of bias batteries in Durafet modules (minimum once per year)

Cleaning of instruments/flumes (biofouling)

Need our own gear for draining/filling inline J-Tubes on the Paragon

Replacement of valves on manual quad valve manifold? (bioflouling)

Inspection of thruster blades.