nsFOCE System Bench Test Plan

08/08/2018

Chad Kecy, Dale Graves

- Main Power

  - Verify GND_ISO and GND_SYS are isolated from one another (resistance)

  - Apply 375Vdc to nsFOCE

  - Verify 24V_SYS, 24V_ISO, and 12V_SYS

  - Verify Digi serial server power

- Ethernet Communication

  - Establish communication with the Wago Coupler

  - Coupler IP Address:

- Power Switching

  - Pump #1 power (12V_SYS)

  - CTD #1 power (12V_SYS)

  - Pump #2 power (12V_SYS)

  - CTD #2 power (12V_SYS)

  - Valve power (24V_SYS)

  - Thruster power (24V_SYS) : Passes 375Vdc

  - pH #1 power (24V_ISO)

  - pH #2 power (24V_ISO)

  - pH #3 power (24V_ISO)

  - pH #4 power (24V_ISO)

- Serial Communication

  - Loopback test to CTD #1 (J8)

  - Loopback test to CTD #2 (J10)

  - Loopback test to thruster (J2)

- pH analog measurements

  - Use a low voltage power supply (0-5V) for analog signal

  - pH #1 (J4)

  - pH #2 (J5)

  - pH #3 (J6)

  - pH #4 (J7)

- RTD measurement

  - Verify proper reading from RTD

- Valve signals

  - Measure output (VALVE_SIG) with voltmeter

  - VALVE_FB (4-20ma signal)

- Sensors

  - Attach (4) pH probes one at a time

  - Attach (2) CTDs one at a time

  - Attach (2) pumps one at a time

- Thruster

  - Verify comms and operation

- Valve

  - Actuate valve and read feedback

- Last