Carbon Wave Glider Software Configuration Considerations for Lab System

List of tunable things:

- Air pump on to circulate ambient air - time

- Air pump on to circulate air through soda lime – time

- Flush system with calibration/span gas – time

- LI-COR on to measure ambient air, zero air, and calibration gas – time

- Acid addition volume – mL or uL (or acid pump on time?)

- Sample addition volume – mL (or sample pump on time?)

- Pump air through acidified sample to LI-COR - time

- LI-COR on to measure sample peak - time

- Air pump on to evacuate sample via backpressure - time

It would be nice to be able to independently control each component (valve/pump) – to test functionality when searching for a failure point.

We could consider including an option to program the water bath temperature to enable running a sequence of experiments across temperature treatments.

List of things to record in experimental file:

- Firmware/software version

- Date/time

- Sample ID

- Calibration gas concentration

- LI-COR measurement output (RH, T, P, xCO2)

- Temperature of water bath