How to change thermistor coefficients and temp offsets

All these parameters are stored under ‘sys_data’ structure in the code.

- Go to main menu -> config

- Edit/Copy EEPROM sys_data structure

  - This allows you to change ANY sys_data parameter. Many of them are very important, so be very careful to only change the parameters you want! It can compromise controller performance and we will have to do a factory reset.

- Thermistor related coeffs are int eh second batch of 13 fields:

  - Second batch of 13 sys_data fields:

  - TCOffset [0.000000]:

  - k0ext [-1.300000]:

  - k0int [-0.400000]:

  - k2ext [-0.001081]:

  - k2int [-0.001455]:

  - quadTherm_c0 [340.98198629984]:

  - quadTherm_c1 [-0.00009102570]:

  - quadTherm_c2 [-95.08806666582]:

  - quadTherm_c3 [0.96537027392]:

  - shTherm_A [0.00210850817]:

  - shTherm_B [0.00007979205]:

  - default_sal [35.000]:

  - sensor_sal [0.000]:

- To update coefficients, just type in the new value. The value in the [ ] is what is currently in memory. Once you type in new coeff, revisit the coefficients to make sure the value you typed in is updated in [ ].

- TCOffset: offset to temperature that is applied AFTER calculating temperature from the quad-term thermistor equation (Honeywell equation).

- 4 terms you wan to change for thermistor: quadTherm_c0 to c3. The default values for Honeywell DuraFETs are in the [ ] .

  - shTherm_A,B,C stands for Steinhardt-hart coefficients (3 parameters). This is not used, but Thom wanted to keep it in in case we wanted to switch to it in the future.

- After entering thermistor coeffs, there will be a lot of additional batches of coefficients (9 batches). Just keep pressing enter and it’ll get you back to the main menu.