Meeting notes Feb 20, 2019

To cover:

- Suspect electrochemistry is happening at reference electrode. How can we deal with this?

- Is it OK for Luke to go through the design?

- Do we have any extra SP boards? I’m missing 2 from the inventory

- Remove water sensor.

- Need 5V supply

  - Curently has 3.45 Switching regulator, then 3.3V linear. This removes any remenants of the switching regulator (high frequency fluctuations).

  - Option 1: use 5V switching regulator (power optode), and then use 3.3V linear. This wastes ~70mW due to voltage loss (~15% of power budget).

  - Option 2: put in extra 5V switching regulator. Space inefficient.

- Chose option 2. Decided to remove water and pressure sensor.

- Header for debugger can go off to the side to save space.

- The board can get longer, not wider

- Move forward with Luke’s design review