Design Requirements for Modular, Miniature Deep-Sea-Durafet

Jan 14, 2019. Document located in Project library/901805/miniDSD

Applications in mind:

- Gliders, Spray and Seagliders (polled)

- LRAUV (continuous/set timing output, a.k.a. data barf)

- BRS (Craig Okuda)

- Other autonomous platforms: Wavegliders, Saildrones

Concept drawing (thanks Gene!):

Must Have

- Good name for this unit

- DSD V2 (designed by Gene)

- 1,000 m pressure rating (Titanium or Al?)

- Check projected life time for all key ICs

- 24 bit resolution (ADS1248) (EOL?)

  - Measures Vrs, Vk, Ik, Ib, Bias Battery V

  - Look into other ADCs

- Current cap adapter circuit + isolated power

- ~1 Hz sampling from wake up to sleep (currently we are around 0.9 Hz)

  - Maybe only measure diagnostics every nth for increased speed.

- Form factor: ~2” OD, ~6” length. Smaller the better (especially smaller diameter)

- Low power (currently ~1 J per measurement), including low sleep power

- Multiple modes of operation: polled, data barf

- Bias battery/power (C-cell? ~1” diameter, 2” length) inside housing. Minimum 1 year. Currently bias current is 250-300 uA.

  - 2 cells are OK.

- External power, RS232 comm (simple command set)

  - Common ground with power, Tx, Rx.

  - Max baud rate: 115,200, but probably will use lower.

- Some sort of pressure relief mechanism

Nice to have (in order of niceness)

- (High priority nice to haves) RTC

- (High priority nice to haves) Memory storage (What size?

- Internal temperature and humidity

- Rechargeable batteries?

- LED blinkers

Should not haves