Deployment details:  Mission ID: 19402901

Shore Station: through UCSD and npzd.  Text based only on npzd

Dates :  Tues 4/2/19 – Fri 5/3/19; 31 days

Dives:  227, max depth 1000m

Deployed and recovered from Paragon

Deployed: 02 Apr 2019 16:53  +36 48.14 -121 51.69

Recoverd: 03 May 2019 17:52  +36 44.93 -122  1.49

Summary:  Spray left Moss and headed out passed M1 where it could do 1km dives.  It then headed North to Francisco Whale to do a coincident CTD cast with the WF.  It then went to 67_80 on line 67 and back to M1 where it met the Paragon for a final 500m CTD cast.

Coincident CTD casts:

WF on 4/10/19 at Francisco Whale

Paragon on 5/3/19 at M1

Issues:

1.  pH parsing error at start of shallow dives

2. differing profile point numbers

3. Weird GPS rollover on dive 35

4. multiple sat packets sent (though I just think this is because session was dicey e.g. sea surface state (edited)

5.  Mechanical offset at 800m for pH?