Deployment details:  Mission ID: 19502901

Shore Station: through UCSD and npzd.  Text based only on npzd

Dates :  5/7/19-5/17/19

Dives:  83, max depth 1000m

Deployed and recovered from Paragon

Deployed: 7 May 2019 17:17 1 +36 47.58 -121 53.44

Recovered: 17 May 2019 16:19 1 +36 44.64 -122  1.92

Summary:  Spray left Moss and headed out passed M1 where it could do 1km dives.  We noticed low voltage on the batteries so turned her around and recovered near M1

Coincident CTD casts:

WF on 5/16/19 close to M2

WF on 5/11/19 close to M2

Issues:

1.  pH parsing error at start of shallow dives

2. differing profile point numbers

3. Low batteries

4. Light intrusion into pH cell, causing pH values to be biased high.