Deployment details:  Mission ID: 19702901

Shore Station: through UCSD and npzd.  Text based only on npzd

Dates :  7/16/19-9/5/19

Dives:  793, max depth 1000m

Deployed and recovered from Paragon

Deployed: 16 Jul 2019 17:24 1 +36 47.82 -121 50.98

Recovered: 05 Sep 2019 15:34 1 +36 47.80 -121 50.49

Summary:  Spray left Moss and headed out passed M1 where it could do a few 1000m dives.  Carson met the Spray and did a simultaneous CTD cast.  Spray then flew M2-67.90-M2-M1-C1.  Spray hung out at C1 for the remainder of the mission

Coincident CTD casts:

July 17 Carson

Issues:

1.  Unstable backscatter

2.  Air vent pump not priming hydraulic pump.  Hydraulic pump recovery required a few times.

3.  Bad pH dives 338, 451, 454, 455, 457, 469, 695.  Most likely a pH hardware issue as it’s microSD card read bad data too.