Quality control and adjustment of biogeochemical measurements

from Spray Glider Observations

January 04 2023

Written by Jacki S. Long (MBARI), Contact: Yui Takeshita (yui@mbari.org)

Monterey Bay Aquarium Research Institute, 7700 Sandholdt Rd., Moss Landing, CA 95039

in Monterey Bay

Quality Flags

As outlined by the GO-BGC Program, quality flags are defined as:

0 – Not QC performed

1 – Good data

2 – Probably good data

3 – Questionable, probably bad data that are potentially adjustable

4 – Bad data

5 – Value changed

8 – Estimated value

9 – Missing data*

*Missing data are filled with ‘NaN’

Quality control

General

The first profile of data are given a QF = 4 (sensors are equilibrating)

For oxygen, pH, and salinity, the deepest 5 measurements are given a QF = 4 to correct for any carry-over effect

Salinity

Data < 30and > 36 are given a QF = 4

Oxygen

Oxygen air-calibrations were performed pre-deployment and glider data are adjusted using the mean of the calculated gains from these measurements.

pH adjustment

To calibrate the pH sensor, a discrete sample is collected next to the glider while the glider also collects measurements for pH. These samples are analyzed spectrophotometrically pre-deployment.

The difference between CANYON-B estimated pH and discrete measurements of pH is minimal between 400m and 500m (Takeshita et al., 2020). Therefore, to account for sensor drift, calibration samples are collected in this depth range throughout the deployment. Drift is determined by comparing the difference between these samples and CANYON-B estimated pH over time, offsets are adjusted profile-by-profile through the full depth.

Data < 7.2 and > 8.5 are given a QF = 4

Fluorescence

The effect of non-photochemical quenching (NPQ), is adjusted to avoid having day-time estimates biased low, for this, we follow the methods presented in Xing et al., 2012.

Estimated variables

Carbonate parameters

Estimated using CO2SYSv3 (Sharp et al., 2021)

pH values > 8.8 and < 7 are set to NaN before running CO2SYSv3

References

Takeshita, Y., Jones, B. D., Johnson, K. S., Chavez, F. P., Rudnick, D. L., Blum, M., et al. (2020). Accurate pH and O2 measurements from Spray underwater gliders. Journal of Atmospheric and Oceanic Technology, 181Ð195. https://doi.org/10.1175/jtech-d-20-0095.1

Xing, X., Claustre, H., Blain, S., DÕOrtenzio, F., Antoine, D., Ras, J., & Guinet, C. (2012). Quenching correction for in vivo chlorophyll fluorescence acquired by autonomous platforms: A case study with instrumented elephant seals in the Kerguelen region (Southern Ocean). Limnology and Oceanography: Methods, 10(JULY), 483Ð495. https://doi.org/10.4319/lom.2012.10.483

Sharp, J.D., Pierrot, D., Humphreys, M.P., Epitalon, J.-M., Orr, J.C., Lewis, E.R., Wallace, D.W.R. (2021, May 20). CO2SYSv3 for MATLAB (Version v3.2.0). Zenodo. http://doi.org/10.5281/zenodo.3950562