Agenda for tomorrow's meeting:

- Can we use Spray34 controller/hardware to test firmware integration and brackets, and still use Spray29 in the meantime?

Yes

- Can we install modem to Spray29?

Currently Spray29 can carry either modem or OCR, not both at the same time.  They share the 10pin bulkhead port feedthru.  To do both we would have to make a Y-cable and even then I need to see if Spray can be ballasted.  The modem would need to ride behind the tail (Uwe Send config) which would require some machining on the polypro tail cone (doable at mbari).  This config would eliminate the DAT capabilities of the Spray modem but I dont think they are needed since Spray will be tracked by the WG.

Still need to install bladder for 29.

- Update on OCR bracket design

  - Can be done in a day and 3d printed at MBARI.  Would want Irene and Yui to approve before manuf (look at SW assembly drawings)

  - Angle of attack is 20degrees. So will be level on the ascent only.

- Nitty gritty details on requirements for communication between Spray-CF2 and mFET

- Data format for OCR, and what to send back?

  - Example data format:

  - SATFI4227\r0.1234\t0.0141\t-0.0038\t0.0113\n

  - Pre-pend space vs post-pend space. Will it break CF2 processing code?

  - For Sat files, only send back PAR data (CF2 memory is main consideration)

- Commands to poll pH, pH+OCR

Similar polling routine as now. BUT:

If(first measurement of ascent)

Send ‘so\n’

elseif(Pres > pres_thres) % pres based on previous measurement

Send ’ss\n’

Else % shallower than pressure threshold

Send ‘so\n’

End

To get data, send ‘sl\n’ for both cases

Does Spray look for any character before turning off mFET? -no.

- For the switch to turn measurement during descent

  - Use state variable to know when it’s descending. If descending, only use ‘ss\n’

- What is the Spray looking for from mFET during polling?

  - At the beginning, looking for # (sample number)

  - Read up to CR/LF

- Documentation on mods for mFET to accommodate 6V for OCR (Projectlibrary)

Polling routine (8s loop)

- Wake up pH

- Send ‘ss\n’

- Go do other things

- Come back a couple seconds later, grab everything by ‘sl\n’

- Then parse the bufstr by looking for ‘#’

- Pack it up for satellite transmission

- Turn off pH

TO operate “34” while we do OCR integration onto “29”

- Move 29 Pcase to 34

- Move 2 wings of 29 to 34 (on the Pcase already)

To operate “29”

- Machine new Pcases

- Fix Wing

- Rebuild hydraulics

Action Items

- JL to look at pitch and roll data. Mainly in the offshore data, and also when it’s doing diel cycles. Look at 21202901

- How much roll/pitch is too much? -> read literature

- Figure out which column is PAR (TM), and the other columns. And YT

- Where to set this up?

  - Need antenna with GPS/Antennae. Previously did it near LRAUV lab.

- TM to make changes to the firmware and update polling commands

- Test firmware in lab without CF2

- Set up system with CF2 “spray 29” controller

- Decide path for P-case

- Sketch out cabling diagram for ‘34’ that is compatible for all OCR, modem, and planktivore (BJ)

- Figure out Tiny schedule (YT)

- Fix 29 asap

Timelines

- Sometime -> Spray34 deployment with Tiny (dependent on Tiny deployment)

- End of Jan: Finish ADV/EC component for pHET-SOSV4

- End of Jan: start intensive bug testing for V4

- End of Feb -> release pHET-SOS V4, for Glider. Ideally freeze code.

- March-Apr -> benchtop testing with CF2, build brackets, etc.

- Early May -> finish benchtop testing with CF2

- May: prepare onshore processing code (during deployment + post processing)

- Reconfigure 34 for OCR (no battery change will save time).

- End of May -> Test deployment with OCR + pH

- June -> deployment for WF cruise (everything until here in 1 battery)

Spray34 has 100 days