MinipHOX Pinout

Bulkhead (6pin) (MCBH-6-MP SS)

Terminate to a 4-pin Harwin  (Data in/out in reference to the microcontroller)

4-pin Harwin header		SubConn wet-matable pin/wire  #

1: (Data IN) --------------------	2

2: (Data OUT)------------------	1

3: -------------------------------	4

Serial Cable (Same as SeapHOx)

Pig tail cable P/N: MCIL-6-FS (2’ pigtail)

Terminate cable to DB9 (Female)

DB9(F)	MCIL-6-FS

2		    Black (pin 1)

3		    White (pin 2)

5		    Green (pin 4)