# Real-time MB-System Data Server Communication Protocol Specification

Rev 0    - Henthorn initial proposal Jan 18, 2018

Rev 0.1 - Henthorn

- Changed III.2.f-g beam numbers - or ids - are now distributed amongst the beam ranges, e.g., [id1,rhoX1,rhoY,rhoZ,id2,rhoX2,rhoY2,rhoZ2,...].

- Added checksum.

Rev 0.2 - Henthorn

- Added Heading to the packet structure.

- Changed the name and clarifying comments.

## I. MB-System Data Server

- Emulate the DeltaT beamformer interface

  - Connect to server socket

  - Read continuously on that socket interface

- Command-line app not specific to QNX

  - Do not use auv-qnx framework classes

  - Linux/Cygwin compatible

- Client/Server model with this application as the server

- UDP communications is adequate but not required

- MB-System server acquires data from multibeam data source, processes beam data, xmit results to clients

  - Acquires data from instrument or beamformer (e.g. Reson 7kCenter)

  - Performs data “post-processing” in real-time

  - Pushes processed data packets out to clients

  - Frequency TBD but at least 1 Hz

- Control messages to this app

  - Connect

    - ACK reply

  - Disconnect

    - ACK reply

  - Change processing options on the fly TBD

  - Specify as needed

- Packet structure - see section below

## II. MB-System Data Client

- Connect to server port

- Receive data packets

- Use data as needed

  - Log

  - Inject into TRN

- Initial client will be auv-qnx driver for simulation

  - Quickest path to begin using the data in TRN

  - Log data in auv-qnx

  - Use existing TerrainAid process

  - Send to TRN via the existing TerrainNavClient interface

    - Modify commsT class to handle data

- Future clients

  - Dorado 2018

    - Updated trn_server

  - Sentry 2020

    - Vehicle data acquired from network data sources

    - Client/Server model may not be needed

    - Sentry control system may simply create a TerrainNav object and call it directly

  - Others TBD

## III. Real-time MB-System Data Packet Structure

- Header

  - Fixed size of 8 bytes

  - Uint32 control

    - 1-byte message type or code

    - Reserved for future

      - E.g., processing options employed

  - Uint32 size of body

- Body

  - Double64 Data timestamp

  - Double64 Vehicle Pos X

  - Double64 Vehicle Pos Y

  - Double64 Vehicle Pos Z

  - Double64 Vehicle Heading

  - Uint32 Number of beams (N)

    - Range 0 to N-1 (not all 512 beams are necessarily represented)

    - Beam[0] = starboard-most processed beam

    - Beam[N-1} = port-most processed beam

  - N x (Uint32,Double64 X, Double64 Y, Double64 Z)

    - Beam[0] ID

    - Beam[0] rhox (forward position from sonar - alongtrack)

    - Beam[0] rhoy (starboard pos from sonar - crosstrack)

    - Beam[0] rhoz (vertical pos from sonar - positive down)

    - Beam[1] ID

    - Beam[1] rhox

    - Beam[1] rhoy

    - Beam[1] rhoz

    - And so on…

    - Beam[N-1] ID

    - Beam[N-1] rhox

    - Beam[N-1] rhoy

    - Beam[N-1] rhoz

  - Uint16 checksum

  - Body size

    - (N + 1) x sizeof(Uint32) + (3N + 5) x sizeof(Double64) + sizeof(Uint16)

    - E.g., on Ubuntu 64-bit 14,382 bytes when N = 512

Uint32 Control

Uint32 Body Size

Double64 Timestamp

Double64 Vehicle X

Double64 Vehicle Y

Double64 Vehicle Z

Uint32 N Ids

Uint32 Id 0

Double64 Id 0  rho X

Double64 Id 0  rho Y

Double64 Id 0  rho Z

:

Double64 Id N-1  rho X

Double64 Id N-1  rho Y

Double64 Id N-1  rho  Z

Header

Body

Uint32 Id N-1

Uint32 Checksum

Double64 Vehicle Hdg

- Assume little-endian architectures for now

  - Straightforward conversions are available