# New Reson App Communication Protocol Specification

Rev 0    - Henthorn initial proposal Jan 18, 2018

Rev 0.1 - Henthorn changed III.2.f-g beam numbers - or ids - are distributed amongst the beam ranges, e.g., [id1,rhoX1,rhoY,rhoZ,id2,rhoX2,rhoY2,rhoZ2,...]. Added checksum.

## I. Reson Beamformer Server (new Caress app)

- Emulate the DeltaT beamformer interface

- Command-line app not specific to QNX

  - Do not use auv-qnx framework classes

  - Linux/Cygwin compatible

- Client/Server model with the beamformer as the server

- UDP communications

- Reson beamformer server acquires data, processes beams, xmit results

  - Acquires data from reson instrument

  - Performs data processing

  - Publishes processed data packets to UDP socket

  - Frequency TBD but at least 1 Hz

- Control messages to Reson app TBD

  - Change processing options on the fly

  - Specify as needed

- Packet structure - see section below

## II. Reson Beamformer Client

- Connect to beamformer server UDP port

- Receive beamformer packets

- Use data as needed

  - Log

  - Inject into TRN

- Initial client will be auv-qnx driver

  - Quickest path to begin using the beamformed data in TRN

  - Log beamformer data in auv-qnx

  - Use existing TerrainAid process

  - Send to TRN via the existing TerrainNavClient interface

    - Modify commsT class to handle data

- Future clients

  - Updated TRN server

    - For use in Sentry 2020

    - Data acquired from network data sources

  - TBD

## III. Reson Beamformer Packet Structure

- Header

  - Fixed size of 8 bytes

  - Uint32 control

    - 1-byte message type or code

    - Reserved for future

      - E.g., processing options employed

  - Uint32 size of body

- Body

  - Double64 Data timestamp

  - Double64 Vehicle Pos X

  - Double64 Vehicle Pos Y

  - Double64 Vehicle Pos Z

  - Uint32 Number of beams (N)

    - Range 0 to N-1 (not all 512 beams are necessarily represented)

    - Beam[0] = starboard-most processed beam

    - Beam[N-1} = port-most processed beam

  - N x (Uint32,Double64 X, Double64 Y, Double64 Z)

    - Beam[0] ID

    - Beam[0] rhox

    - Beam[0] rhoy

    - Beam[0] rhoz

    - Beam[1] ID

    - Beam[1] rhox

    - Beam[1] rhoy

    - Beam[1] rhoz

    - And so on…

    - Beam[N-1] ID

    - Beam[N-1] rhox

    - Beam[N-1] rhoy

    - Beam[N-1] rhoz

  - Uint32 checksum

  - Body size

    - (N + 1) x sizeof(Uint32) + (3N + 4) x sizeof(Double64)

    - 14,372 bytes N = 512

Uint32 Control

Uint32 Body Size

Double64 Timestamp

Double64 Vehicle X

Double64 Vehicle Y

Double64 Vehicle Z

Uint32 N Ids

Uint32 Id 0

Double64 Id 0  rho X

Double64 Id 0  rho Y

Double64 Id 0  rho Z

:

Double64 Id N-1  rho X

Double64 Id N-1  rho Y

Double64 Id N-1  rho  Z

Header

Body

Uint32 Id N-1

Uint32 C

- Assume little-endian architectures for now

  - Straightforward conversions are available