| Energy Use Each Image |  |
| --- | --- |
|  | Energy per image (WHr) |
| Canon EOS 5D MarkIIII | 0.02 |
| Strobes (2) | 0.446 |
| SUM= | 0.466 |
| Hotel power |  |
|  | Energy per year (WHr) |
| Snappy Controller | 17 |
| Total Energy per year at one image per hour (WHr) |  |
| Canon + strobes | 4082.16 |
| Snappy | 17 |
| Sum= | 4099.16 |
| Li pack derated= | 4.2 kWHr ->8975 pics (8760 pics/1 yr |

These calculations above were done for the Smith tripod which has proved to work for 1 year in the field repeatedly.

For DiSCO we may start with Alkaline packs, the Smith tripod alkaline pack  is 19 D cells per layer, 7 layers per pack. This pack is >2 inches longer than the Lithium pack. If we made a pack with 5 layers it would fit.

The existing Alkaline pack would yield about:

19 cells x 7 layers = 133 D cells

One D cell is 14 AHr x 1.2 V= 16.8 WHr

Derated for temp by 30% = 11.76 WHr ( extremely conservative, worst conditions probably better than this)

One pack (133 cells) = 1.564 kWHrs

Given 0.466 WHr/image, and subtracting 17 WHr for Snappy (conservative because that is for a year):

(1564 WHrs -17 WHrs)/0.466 WHrs/image = 3319 images

A partial pack missing one layer would yield (6/7)*(1564-17)/0.466 = 2845

A partial pack missing two layers would yield = 2370 images  -> 98 days