Battery pack from Electrochem pn. 3PD1848

Pack is 8S1P, cells are CSC93 DD (p.n. 3B0036)

Voltage per cell =  3.25 V

Pack voltage = 3.25V * 8 cells = 26V

Capacity = 26 AHr

Total energy in pack = 26 AHr * 26 V = 676 WHr

Derate for cold 15% = 676 WHr * 0.85 = 575 WHr

Total images per pack at 1 image/hour = 575 WHr/85 mWHr = 6764 images