- Right Click where it says EPOS 2 under the COM port and select “Connect”. This will take a few seconds and you will see the software talking to the motor controller

- Select the tab that says “I/O monitor”, this is where you will turn the limit switches on.

  - The Positive (or lower limit switch) is Digital 1

  - The Negative (or upper limit switch) is Digital input 2

  - Enable each digital input under the column “Exec Mask”, using the pull down menu.

- Select the tab that says Velocity Mode

  - Press “Activate Velocity Mode in the upper left

  - Set the velocity to 500 rpm under “Setting Value” Remember positive tilts the camera down and negative tilts the camera up

  - Then in low left hand corner clicke on “disabled” to enable the drive

  - Before you start moving know that you can stop the motion by pressing “Quickstop”

  - To move the motor click on “Apply the setting value”

  - You may want to arrange your windows on the laptop so you can see the liveview and the motor controller software at the same time.

  - Stop when you want by pressing quickstop.

  - To start again you will need to press “Quickstop” again and then press apply setting value.

  - If you trigger a limit switch you need to go into the lower status window and right click on the error and select “Clear All Errors”

  - When you are happy with the position disable the motor and then close the EPOS software.