Deepsea Coral Observatory Functional Requirements

Far field Imaging System

Functional Requirements:

- The DiSCO shall image a fixed area of a chosen coral community. The area should be large enough to be relevant to abundance studies.

- The DiSCO shall be capture images of substantial resolution to identify 10 cm organisms.

- The DiSCO shall be engineered to operate without maintenance for up to one year.

- The DiSCO shall be deployable at sites as deep as Sur Ridge. 4k preferable.

- The DiSCO shall capture images at the frequency necessary to study changes in community structure. Every hour sufficient.

- The DiSCO shall be deployable as a free vehicle.

- The DiSCO shall leverage to the extent possible from previously developed technology and COTS solutions.

- The DiSCO shall be operated and maintained by non-engineering personnel.

- The DisCO shall be deployable from the Western Flyer. Nice to have Rachel Carson.

Design Requirements:

- Imaging requirements:

  - Area to be imaged: 20 m2 [FR1]

  - Resolution requirement:  1 cm [FR2]

  - Imaging frequency: 1 per hour[FR5]

  - Color required: yes [FR1]

  - Laser(s) to help with positioning – requirement [FR1]

  - Exposure: auto or fixed

- Environmental Requirements:

  - Depth: 2000 m [FR4], 4000 m nice to have

  - Current: 1 knot [FR4]

  - How far off is the target:

  - How high off the seafloor is the target:

  - Restriction on drop weight : in the sanctuary so can only leave if over 500 lbs[FR4]

- Mechanical Requirements:

  - Must be able to position camera in front of area of interest. [FR1]

  - An ROV must be able to manipulate and carry the DiSCO. [FR1]

  - Must weigh little enough and have sufficient righting current to be moved by ROV [FR1] but enough not to be moved by 1 knot current [FR4]

  - Must have an acoustic release [FR6], would be nice to design for 2 releases but ok to only have one.

  - Must have manual release.

- Electrical Requirements:

  - Instrument must include a battery sufficient to operate the system for a year taking images at the desired frequency.[FR3] [FR5]

  - The instrument must be able to control and synchronize camera and lighting.

  - Power to the  positioning lasers must be controllable. [FR1]

  - The instr

- Design Derived Requirements:

  - Camera must be X meters above the seafloor

  - Camera must have X resolution

  - Illumination required sufficient to light up a field of X by X meters

  - Data storage required for X images at X MB per image

  - Must be autonomous, battery operated

  - Lasers must be enabled at start of mission and disabled after X hours

- Desired features for future upgrades

  - Ability to tilt camera