Procedure for downloading and installing 4K license key for HDC-P50 camera.

- Purchase HZC-UHD50 license option from Sony.

- Go to website: https://ulms.sony.net/portal/ and create a user account if you do not already have one.

- After login, enter the purchase code received from Sony vendor. Use no dashes or spaces, often best to copy and paste code directly into the website.

- Website should verify the code entry and the purchased license part number and show the available number of licenses.

- Click on “install license” FOR THE HDC-P50 camera in the list of cameras and then “download installation key” and you should receive a file download named “RQ_LIC.DAT”.

- Place this file in the root directory of a USB drive and install in the camera using the camera instructions for installing a key.

- After installation and a power cycle of the camera, you should see that all of the 4K camera settings appear in the camera menus.

Mark Chaffey 2/25/2020