| WORK ORDER LABEL HERE | Optim MxD Empty Housing Assembly Inspection Worksheet | 741-000-0070-00-0A Doc. Ver. 6.2 For Assembly: 741-000-0056-00-0A Serial Number: 41-000-_________ |
| --- | --- | --- |
|  | Product Description: | OPTIM MXD, EMPTY HSG, 4KM, MINF-1FO-RA MINK-19FCR-RA MINK-10FCR |
|  | Sales Order #: | DO141065 |
|  | Build Start Date: | 02/17/2023 |
|  | Build Complete Date: | 05/31/2023 |
|  | Assembler: | H. Duran / J. Buss |
|  | Inspector: | A. Steiner |
|  | Drawing P/N Referenced: | 741-000-0130-00 |
|  | MAI P/N Referenced: | N/A |

|  | Pre-Assembly |  |  |  | ASSBLR | INSPCTR |
| --- | --- | --- | --- | --- | --- | --- |
|  | Documentation | Record the heat number (receiver ID, if glass dome) of the following parts: |  |  | H.D. J.B. |  |
|  |  | Part Number | Heat Number/Receiver ID |  |  |  |
|  |  | 741-000-0026-02-0A (housing) | PVD7235 (039791) |  |  |  |
|  |  | 741-000-0004-02-0A (dome seat ring) | A13767 |  |  |  |
|  |  | 741-000-0005-00-0A (tapered ring) | A13767 |  |  |  |
|  |  | 741-000-0008-01-0A (seal retainer) | WOF60 |  |  |  |
|  |  | 741-000-0007-01-0A (aft flange) | WOF60 |  |  |  |
|  |  | 741-000-0054-00-0B (endcap) | PVD6080 (040292) |  |  |  |
|  |  | 741-000-0013-01-0A (endcap retainer) | WOF60 |  |  |  |
|  |  | 772-013-0000-03-0A (coated glass dome) | DOME #11, DP147669 |  |  |  |
|  | Part Condition | Sealing and load bearing surfaces are free of scratches, markings, dents, dings, and any visible defects. |  |  | H.D. |  |
|  | Assembly |  |  |  | ASSBLR | INSPCTR |
|  | The housing assembly is clean and free of extraneous grease, residue, debris, or dirt. |  |  |  | H.D. |  |
|  | All parts of the assembly are securely held together without any movement. |  |  |  | H.D. |  |
|  | Endcap sealing surface is free of scratches, markings, dents, dings, and any visible defects. |  |  |  | H.D. |  |
|  | Check all screws holding down the forward dome retainer are tight and that there is no gap between the forward and aft port retainer rings. |  |  |  | H.D. | J.B. |
|  | The assembly has been inspected by a manager before proceeding to the pressure test portion. |  |  |  |  | A.S. |
|  | Pre-Pressure Test Oil Bath Inspection: Glass dome surfaces and seat are undamaged. |  |  |  | H.D. |  |
|  | Proof Pressure Test | Proof pressure test: 1 cycle for 15 minutes at 7,250 psi (4km) |  | Start Date: _04/27/23_______ Log Book #: 20-23-01 Page #: _054_____ Result (circle one): Pass / Fail | (This information is to be completed & initialed by the technician performing the test.) Initial: _K.F._________ |  |
|  | Post-Pressure Test Oil Bath Inspection: Glass dome surfaces and seat are undamaged. |  |  |  | H.D. |  |
|  | Dome Seat Inspection: Inspect the condition of the dome seat |  |  |  | H.D. |  |
|  | Dome inside apex to chassis stop face depth: Measure the dome and support ring stack depth measurement and record values below. |  |  |  | H.D. |  |
|  |  | Nominal Per-Spec |  | As Measured |  |  |
|  | Dome Internal Apex | 75.81 mm |  | 75.92mm |  |  |
|  | Compliance Film | 0.13 mm |  | 0.13mm |  |  |
|  | Seat Ring | 14.00 mm |  | 13.99mm |  |  |
|  | Tapered Ring | 58.50 mm |  | 58.30mm |  |  |
|  | Sum | 148.44 mm |  | 148.34mm |  |  |
|  | Delta (Nominal & as-measured) |  |  |  |  |  |
|  | Pressure test (see Appendix 4.1): 10 cycles for 1 minute at 6,380 psi (4km) THEN 1 cycle for 60 minutes at 6,380 psi (4km) |  |  | Start Date: _04/27/23_______ Log Book #: 20-23-01 Page #: _054_____ Result (circle one): Pass / Fail | (This information is to be completed & initialed by the technician performing the test.) Initial: ____K.F.______ |  |
|  | Post-Pressure Test Oil Bath Inspection: Glass dome surfaces and seat are undamaged. |  |  |  | H.D. |  |
|  | Dome Seat Inspection: Inspect the condition of the dome seat |  |  |  | H.D. |  |
|  | (POST-TEST) Dome to support ring centration: Measure the centration of the dome to support ring using the DAQ/LVDT setup. Print the “Report” tab and include it with this inspection checklist. Record the average dome decenter vector to the right. NOTE: Power up and allow LVDT instrumentation to stabilize for 10 minutes MINIMUM before taking measurements! |  |  | ________0.05mm____ _______<6°_____ LIMIT: ¦0.424mm¦ Record waiting time here: ____________ minutes | H.D. |  |
|  | Low Pressure Vacuum Check: Without dummy plug installed, draw vacuum on the assembly of 29” of mercury for a MIN of 2 hours. Record the change: _____________” |  |  |  | H.D. |  |
|  | HIPOT connector to housing (500VDC). |  |  |  | N/A |  |
|  | Packaging/Storage |  |  |  | ASSBLR | INSPCTR |
|  | Housing sub-assembly is stored in a protective and enclosed foam-lined box. |  |  |  | H.D. |  |

- APPENDIX

  - Pressure Test Profile Chart