Display Recommendations for MBARI 4K Video

Fellow MBARIans,

We are in the process of starting a transition to capturing video on our ROV’s in the ultra-high definition 4K format (UHD 4K). In addition to four times the resolution of our current high definition format, the UHD 4K offers a wider color space and more dynamic range between the dark and light areas in an image.

In order to take full advantage of what the UHD format offers, your video display must support the format in the following two ways:

Hybrid Log Gamma or HLG. We plan on capturing images in the HLG specification. If your display can be set for an HLG signal you will see significantly brighter and more realistic highlights in the video image. If your display cannot be set for HLG you will still see a great picture but not the full dynamic range of HLG reproduction. In other words HLG is backward compatible with non-HLG capable displays.

BT.2020 colorspace (ITU-R Recommendation BT.2020). We would very much like to capture images in the new standard for color representation known as BT.2020 or Rec.2020. This standard for color capture has the ability to reproduce a larger range of colors visible to the human eye than the traditional Rec. 601 or Rec. 709 display color space that dates back to cathode ray tube displays. The result is more realistic color reproduction. Unfortunately, BT.2020 is not directly backward compatible with non-BT.2020 displays, and will look undersaturated on these displays with colors that are dull and washed out. This may be OK for some applications but for critical viewing it is unacceptable. The current implementation plan is to survey the displays that will be used for video observations and if a significant number of displays will support the BT.2020 format then we will capture in this format. If there are not enough BT.2020 displays available by the end of 2020 we can elect to initially capture video in the Rec. 709 color space and transition at a later date.

RECOMMENDATION FOR ANY CURRENT AND VIDEO DISPLAY PURCHASES

If you plan on working with 4K ROV video at MBARI and want to have the best image reproduction possible it is recommended that any future UHD display purchases (including computer display monitors) support these standards:

Resolution at least 3840x2160 (commonly referred to as “UHD”)

Can display 4K at 60 (59.94) frames per second, i.e. the 2160 P60 frame rate

Supports HLG (Hybrid Log Gamma)

Supports Rec.2020 colorspace

Here is a short list of example monitors. We will update this during the year.

Displays for live video (ROV control room):

Most Boland HDR series monitors (check color space specification, some support HLG but do not support Rec.2020), various sizes, input: video 12G SDI on BNC connector.

High end displays for video annotation, editing and grading. These displays can be color calibrated to provide true color reproduction:

Apple Pro Display XDR, 32” diagonal, input Thunderbolt 3 (USB-C), $6000

Most Boland 4K HDR series monitors (check color space specification, some do not support Rec.2020), various sizes, input: video 12G SDI on BNC connector.

ASUS ProArt PA32UCX, 32” diagonal, input: HDMI and Thunderbolt 3 (USB-C). $4000

Sony PVM-X2400. $10,000

Medium quality computer displays. For computer displays, the graphics processing takes place in the computer graphics card (GPU) and the operating system. The video displayed is mapped to the properties of the video monitor by the GPU and the higher the quality of the display, the wider the brightness range and color reproduction. At a minimum, the display should support high dynamic range (HDR) and a “wide” color gamut.

Windows 10 computers:

https://support.microsoft.com/en-us/help/4040547

Apple computers:

Apple Pro Display XDR, 32” diagonal, input Thunderbolt 3 (USB-C), $6000

and televisions. For a television with a 4K video input that does internal graphics processing, they should support HLG and Rec.2020:

Dell Computer – they have no HLG compatible displays.

LG manufactures many TV displays and computer monitors that claim to be “Ultra HD”.

https://displayhdr.org/certified-products/