MiVEGAS TAILCONE FINISHWORK						5/23/16

- Grind out a 0.05” relief on reinforced areas at the side fin shaft holes, room for spacers

- Enlarge hole to oval shape for antenna wires, use shoe for guide

- Enlarge holes (4) for ant mast mount to .23”, easier fitting.

- Make slot for ¼-20 aft ant mount.

- Match drill 0.09” hole for Argos ant pass thru

- Abraid area aft of antenna hole for Argos radio to be bonded to inner shell, use flexible epoxy

- If necessary, Slightly enlarge all 8 mount holes for tailcone

- Seal all abraided, cut  areas to seal C/F.  low viscosity Epoxy

- Reinforcing guide strips and screw mounts of G-10 may be bonded on as well