Mass and inertia data (kg, m units) for:

- Nose section (short nose with std sensor payload)

Mass of 16.77 kg, centered at 1.126, 0.003, 0.004

Principal axes of inertia and principal moments of inertia: (kilograms * square meters)

Taken at the center of mass.

Ix = ( 1.000,  0.011,  0.007)   	Px = 0.239

Iy = (-0.008,  0.073,  0.997)   	Py = 0.516

Iz = ( 0.010, -0.997,  0.073)   	Pz = 0.537

Moments of inertia: ( kilograms * square meters )

Taken at the center of mass and aligned with the output coordinate system.

Lxx = 0.239	Lxy = 0.003	Lxz = 0.002

Lyx = 0.003	Lyy = 0.537	Lyz = 0.002

Lzx = 0.002	Lzy = 0.002	Lzz = 0.516

Moments of inertia: ( kilograms * square meters )

Taken at the output coordinate system.

Ixx = 0.240	Ixy = 0.065	Ixz = 0.082

Iyx = 0.065	Iyy = 27.357	Iyz = 0.002

Izx = 0.082	Izy = 0.002	Izz = 27.336

- Tail-cone section (without DW, fins, prop):

Mass of 16.35 kg, centered at  -0.277, 0,0.004

Principal axes of inertia and principal moments of inertia: ( kilograms * square meters )

Taken at the center of mass.

Ix = ( 1.000,  0.001,  0.022)   	Px = 0.210

Iy = (-0.022,  0.014,  1.000)   	Py = 0.767

Iz = ( 0.000, -1.000,  0.014)   	Pz = 0.853

Moments of inertia: ( kilograms * square meters )

Taken at the center of mass and aligned with the output coordinate system.

Lxx = 0.210	Lxy = 0.000	Lxz = 0.012

Lyx = 0.000	Lyy = 0.853	Lyz = 0.001

Lzx = 0.012	Lzy = 0.001	Lzz = 0.767

Moments of inertia: ( kilograms * square meters )

Taken at the output coordinate system.

Ixx = 0.210	Ixy = 0.000	Ixz = -0.006

Iyx = 0.000	Iyy = 2.104	Iyz = 0.001

Izx = -0.006	Izy = 0.001	Izz = 2.018

- Pressure hull section (without the battery pack)

Mass of 44.6 kg, centered at 0.457,0,-0.003

Principal axes of inertia and principal moments of inertia: ( kilograms * square meters )

Taken at the center of mass.

Ix = ( 1.000,  0.000,  0.003)   	Px = 0.708

Iy = (-0.003, -0.037,  0.999)   	Py = 8.237

Iz = ( 0.001, -0.999, -0.037)   	Pz = 8.262

Moments of inertia: ( kilograms * square meters )

Taken at the center of mass and aligned with the output coordinate system.

Lxx = 0.708	Lxy = 0.004	Lxz = 0.019

Lyx = 0.004	Lyy = 8.262	Lyz = -0.001

Lzx = 0.019	Lzy = -0.001	Lzz = 8.237

Moments of inertia: ( kilograms * square meters )

Taken at the output coordinate system.

Ixx = 0.709	Ixy = 0.002	Ixz = -0.037

Iyx = 0.002	Iyy = 17.565	Iyz = -0.001

Izx = -0.037	Izy = -0.001	Izz = 17.540

- X, Y, Z positions (in meters) referenced to the rear connector lug face origin for:

  - Battery pack (centered):  31.8 kg, centered at 0.563,0,0.024

  - Drop weight:  -0.128,0,0.142,

  - VBS external bladder -0.137,0,-0.045