ABSTRACT FORM

(Please note: The complete abstract should be limited to three pages, if possible.)

Project Title: LRAUV Multi-vehicle simulation environment for LRAUV

Expected Project Duration (in years): 3

| PI | Project Manager | Lead Scientist | Lead Engineer |
| --- | --- | --- | --- |
| Ben Raanan | Ben Raanan | Yanwu Zhang | Brian Kieft |

(Indicate individuals appropriate for your project. Not all projects need all four positions filled in.)

Project Description:  Multi-vehicle operations are essential to the future of autonomous vehicles in ocean science. MBARI is already a leader in the application of multi-vehicle operations in ocean science, and we are uniquely poised to advance these operations and the scientific research they enable. Alongside MBARI’s growing fleet of LRAUVs, adding appropriate tools for development and validation of new applications will be crucial for scaling up multi-vehicle operations and pushing the limits of autonomous ocean research and discovery. Adding to our rogueries field-testing practices, the ability to simulate and visualize a multi-vehicle scenario under realistic and reproducible settings will enable rapid prototyping and testing of new algorithms, and improve the overall reliability, extensibility, and exportability of the LRAUV platform.

To this end, we propose a three-year effort in which we will collaborate with the team at Open Robotics to integrate LRAUV’s software ecosystem with their open-source framework for robotics and multi-robot simulation.

In addition to the project description above, please provide one (1) paragraph addressing each of the following topics:

Fundamental Problem

What’s the grand challenge this work addresses? Why should we care? What makes this work relevant to MBARI?

Multi-LRAUV collaborative methods have become a key component in MBARI field programs such as CANON (Chavez), ESP (Birch and Scholin), Targeted Sampling (Zhang), Terrain Relative Navigation (Rock), Acoustical Ocean Ecology (Benoit-Bird), and Biodiversity & Biooptics (Haddock). The success of multi-vehicle operations at MBARI is the result of diligent design work and meticulous field testing by science and engineering staff. Looking ahead, we will need to evolve an effective and scalable development path to support the increasing demand and growing complexity of future multi-vehicle applications, and must do so in a way that is extensible, limits operational overhead, and optimizes for tech transfer.

The LRAUV software stack currently includes a faster than real-time sim, which covers everything from the mission/guidance level down to vehicle controls and dynamics, but lacks real-time visualization capabilities and is limited to a single-vehicle. More recently, we’ve added a network-based messaging interface using LCM middleware that enables telemetry and “backseat” control over the vehicle from other processes, including apps that execute on a separate/secondary processing unit mounted in the hull. These two features are complementary, as they allow researchers to develop implementations independently offline and then upload them to the LRAUV backseat computer for execution.

Through collaboration with the team of simulation experts at Open Robotics (https://www.openrobotics.org), we have an opportunity to catalyze the development of a fully visualized multi-vehicle simulator for LRAUV and extend LRAUV’s existing backseat architecture using common interfaces that are widely adopted in the open-source robotics world, including in the underwater vehicle community. The software tools and vehicle interfaces that we will develop will enable researchers to quickly iterate and validate the performance of their multi-vehicle algorithms and backseat apps, and overall, make development for the LRAUV more accessible to researchers and collaborators seeking advanced methods of ocean observation.

Technology

What engineering & technology is needed to move forward (e.g., sensor, platform, ship, de novo R&D)?

We will create a multi-vehicle underwater simulator and software interfaces for LRAUV based on Robot Operating System (ROS) and its robot simulation and visualization tool Gazebo, and integrate them with the existing LRAUV software stack.

ROS is the industry standard for development in most robotic domains and has been widely adopted in the marine robotics community, including by many of MBARI’s collaborators (NPS, WHOI, Liquid Robotics). Gazebo is a 3D dynamic simulator capable of accurately and efficiently simulating populations of robots in complex (underwater) environments. Gazebo offers a robust physics engine, high-quality graphics, and convenient programmatic and graphical interfaces. ROS and Gazebo are both open-source with hundreds of thousands of active monthly users who regularly contribute to its open repositories.

Open Robotics is the main developer and supporter of ROS and Gazebo, and their extensive experience using this software and creating open-source frameworks will guarantee the success of this project. Open Robotics will be contracted to develop the following software components in support of this effort, which can be divided to into 3 phases:

- A Gazebo vehicle model for LRAUV: port the vehicle’s kinematic and dynamic attributes, surface properties, textures, and on-board instrumentation, including actuators and sensors plugins, to a Gazebo compatible format. 
Result: the resulting LRAUV vehicle model can be hosted by any Gazebo simulation, and will be drivable via standard ROS message interface (Figure 1).

Figure 1: Gazebo vehicle model example: ECA Group's A9 AUV.

- A Gazebo world model: construct a simulated underwater environment with a digital elevation map of a section of Monterey Bay’s shelf and extension points for adding current, wind, and wave models. 
Result: the world model will be supported by Gazebo’s built in physics engine, fully rendered in an interactive 3D environment (Figure 2), and will be capable of hosting multiple vehicle models in conjunction. Simulated vehicles and sensors interact with rendered environment (e.g., collisions, DVL beam raytracing). As part of this phase, we will make use of previous work by our 2019 summer intern to incorporate simulated acoustic messaging for multi-vehicle coordination.

Figure 2: sample Gazebo world showing modelled ocean surface (top), and an ECA A9 model underwater over a realistic elevation map of the seabed (bottom).

- Full integration with LRAUV’s software stack: bridge LRAUV data formats for compatibility with the ROS message passing interface, and synchronize the Gazebo sim with LRAUV application processes to allow full simulation with the vehicle’s software stack in the loop, including mission/behaviors, controllers, and device drivers.

Data

What data will be generated? How will they be managed? Visualization and IP considerations?

1 GB of data stored on Atlas server.

Intellectual Property (IP) Considerations

Do you anticipate a need for a cooperative research and development agreement or licensing arrangement with an external entity? Will the work potentially result in a patentable product or service? Will novel IP be created, and how might it be disseminated (e.g., license agreement and/or collaborations)?

For additional guidance please review MBARI’s intellectual property internal web page: https://mww.mbari.org/itd/ip/.

During development we will work with Open Robotics under a collaborators agreement and NDA to figure out what aspects of the projects can be open-sourced.

Anticipated Impact

Who are the consumers of the instruments, methods, technology, and data that will be generated within and beyond MBARI? Will the project create tangible connections with other organizations (e.g., academic, NGO, MBA, MBNMS, state or federal agency)?

[Click here to enter text]

Anticipated Outreach Needs

Thinking about the audiences/consumers you highlighted in the question above, do you anticipate needing to create outreach assets to help increase your impact? If so, please specify whether you anticipate needing to work with ITD and also specify what type of asset (graphic, animation, video, etc.) you might need.

None

Please provide an overall, high-level estimate of resources envisioned for this project in 2020 to help give a sense of the size of the proposed effort. Detailed scoping is not needed at this stage. Simply mark the appropriate box below.

| Budget Category | Minimal | Medium | Significant |
| --- | --- | --- | --- |
| Labor | x |  |  |
| Expense |  | x |  |
| Ship/Assets |  |  |  |

If you anticipate a large ramp-up or increase in resource requests in future years of the project, please include that information below.

Tiered approach for spending