Testing Plan, Rev2c

AUV Docking on RASP

List of Milestones:

- Demonstrate acoustic homing hardware (DAT)

- Demonstrate AUV homing to a moored transponder

- Add acoustic telemetery to relay dock heading to AUV

- Demonstrate AUV an home towards transponder along a prescribed heading

  - Video image of AUV hitting a subsea target (mock dock)

- Add high-update homing

- Dock AUV into stationary dock

- Add go-around or abort capability based on the vertical motion of the dock.

- Move dock to buoy, demonstrate in calm seas

- Add latch and power/data transfer system

- Demonstrate active stabilization of dock using PTO system

- Demonstrate  AUV docking in Sea State X

Test 1: DAT Validation, MS#1 –  July 11th - DONE

This test is designed to validate the basic functionality of the DAT acoustic homing system and the AUV’s DAT interface and logging system.  We will also explore the near-surface performance based on the ratio of valid range/bearing fixes to the number of interrogations at various depths between 5 and 30 meters deep.  We will integrate the (un-calibrated) DAT into the AUV and operate the system offshore Moss Landing.  The AUV will navigate on its own and log the range/bearing/elevation data output by the DAT system while making repeated open-loop runs toward a transponder suspended at various depths below a small boat.

Test 2: Pure Pursuit Homing on Mock Dock, MS#2 – Week of Oct.  17th, earlier if AUV is available.

During Test 2 we will demonstrate the capability of AUV to autonomously home to a seafloor mounted transponder from an arbitrary heading.  This second test doesn’t require the fast-update rate DAT upgrade or acoustic transmission of the dock heading.  The main task for this test is to port the Dorado-based homing code to the Tethys code-base and test at sea.

Control Algorithm and Code: We will port the homing code from Dorado to Tethys, including navigation reset and homing offset.  Make necessary adjustments to use the DAT instead of the Sonardyne.  Simulate homing prior to Test 2, which requires a simulated DAT.  Conceptually this is easy, but there will be some coding effort to pass the data through the new simulation architecture.

Test 3: Directional Homing 1, MS#3 &4 – Week of October 17th

Test 3 adds the ability for the AUV to home towards the Mock Dock along a prescribed heading.  This test will require Cross-track homing that is analogous to the final approach of an aircraft landing where the aircraft may have to crab into the wind or current to maintain a constant course aligned to the runway.  The depth will be hard-coded for this demonstration of 2-D homing

Control Algorithm and Code: Verify cross-track control using the DAT instead of the Sonardyne.  This may require modifying or redesigning the median filter.  Test in simulation prior to Test3.

Add high-rate update mode.

Test 4: Directional Homing 2 – Week of November 14th

Test 4 adds the complexity of data telemetry between the Mock Dock and the AUV.  The magnetic heading of the dock will be sent to the AUV through the acoustic modem, which is a component of the DAT homing system.  The AUV will then home towards the dock using pure pursuit until approximately 100m away, then switch to cross-track control to match the vehicle’s heading with the Mock Dock’s heading.  If the AUV is within 1.5m of the transponder, it will hit a soft net target and the accuracy of that hit will be recorded by a video camera mounted behind the target.

Control Algorithm and Code:  Make adjustments as necessary. Vehicle switches from pure pursuit to directional at ~100m range.  The previous algorithm was for a fixed-heading dock.

Test 5: Directional Homing 3 – November 21st

This is a repeat of Test 4.

Test 6: Buoy Integration Test 1 – Mid February

The homing transponder and cameras will be bolted to the buoy system during a normally scheduled test in calm weather.  The AUV will perform homing maneuvers with an offset to minimize crashing into the buoy, cable or winged plate.

Control Algorithm and Code:  Change code to home at a fixed depth rather than a fixed altitude.  Add new code to detect unacceptable vertical dock motion, and go around if necessary.  Given a DAT elevation accuracy of 2 degrees, the vehicle will have to be within 30 meters to detect 1 m of vertical motion.  Consider what can be done on the buoy to reduce dock motion during final approach, e.g. actuate PTO to minimize vertical motion of dock and possibly control AUV to also minimize vertical motions.  The vehicle will

Test 7: Docking Test 1 – Mid May

The prototype docking cone and tube will be fitted to the buoy system and tested  during calm weather.

Test 8: Docking Test 2 – June

Repeat of Test 1 in rougher sea conditions

Test 9: Docking Test 3 – July/August

Final test to demonstrate AUV docking