PB Docking Update: 12 January, 2012

- Status

The AUV can query the dock’s heading, and seems to be able to repeat ably home to within a 2m cluster, though so far that cluster is about 2m the right of the target.

  - AUV

    - Tethys AUV is fully operational with the docking hardware (DAT/Camera) and Daphne is fitted with the DAT but hasn’t been tested.  We are planning to test Daphne during the week of January 16th

    - We’re pretty sure the Rotation calibration issue with the DAT on Tethys is resolved.  We know how to address this now, so it shouldn’t take more than a day to correct Daphne’s unit.

    - There is still a problem with DAT Elevation on Tethys.  We are going to collect a sound velocity profile next Monday and Benthos will analyze the acoustic properties of sound channel to see if there is preferred sound propagation channel in the upper water column.  This may also be a calibration offset issue.

    - There are still issues with the cross-track approach and we haven’t let the AUV choose it’s approach heading on its own, e.g. The vehicle gets the dock heading, tells shore, then shore tells the AUV what heading to approach the target.

    - Oneway (fast mode) mode seems to work, but hasn’t been tested at sea

    - We’re having trouble with Tethys’ navigational accuracy: ~ 10% of distance traveled.  The compass calibration seems good, so we suspect the DVL is under-reporting our speed.  This is contributing to errors in plotting, but we don’t think is effecting our homing accuracy.  We’ll see how Daphne’s navigation looks next week.

  - Mock Dock

    - The Mock Dock is operational, though with reduced capabilities.

      - We are able to power and communicate with the acoustic modem on the dock

    - We are still having RF interference issues with the telemetry between MBARI and the dock.  A set of 2.4 GHz radios are being shipped down to temporarily replace the 900 MHz radios.

    - The light on the dock is not working, though we have a wiring change ready to implement on Tuesday that should get it working again.

    - One of the two batteries has been disconnected because it is flooded with seawater

    - The buoy to dock telemetry modems stop working for approx. 15 seconds every minute.  I suspect electrical noise interference or a disruption of power that causes it to reboot.

    - These two telemetry disruptions (buoy/dock and shore/buoy) have rendered the dock camera un-usable.

    - We fitted a camera to the AUV to document targeting.  In mid-day we don’t need a light at 35m depth, but as the water clarity changes and we perform more targeting runs earlier and later in the day we need a light.  We did some runs with a small light on the AUV, but this wasn’t adequate so we’re working to get the light on the dock working again.

  - Real Dock

    - Manufacturing of the AUV dock has begun following several Critical Design Reviews

    - The electrical design is progressing, but much work is still required

- Next Steps:

  - Next week:

    - Fix the light on the dock so we get good video of targeting

    - Work on the navigational accuracy problem.  Run a box mission to separate the compass from the speed measurement

    - Fine-tune the Rotation calibration number to hopefully move the homing to the center of the target

    - Solve the DAT elevation problem

    - Improve the Short/Buoy telemetry link

  - Over next month:

    - Allow AUV to do cross-track homing based on dock heading, with Oneway mode for the last 50m.

    - Gain confidence we can accurately home to a fixed dock

    - Get good video of the AUV hitting the dock.

    - Recover the Mock Dock

    - Upgrade the dock to support the Real Dock with Buoy electronics

- Milestones

  - Complete Mock Dock operations

  - Deploy stand-alone Real Dock

  - Demo docking, latching, comms/power transfer

  - Integrate on Power Buoy

  - Test 1

  - Test 2