PB Docking Update: 2 June, 2011

- Group focus is on the July 11 DAT Validation tests

  - Test 1: DAT Validation –  July 11th.  This test is designed to validate the basic functionality of the DAT acoustic homing system and the AUV’s DAT interface and logging system.  We will also explore the near-surface performance based on the ratio of valid range/bearing fixes to the number of interrogations at various depths between 5 and 30 meters deep.  We will integrate the (un-calibrated) DAT into the AUV and operate the system offshore Moss Landing.  The AUV will navigate on its own and log the range/bearing data output by the DAT system while making repeated open-loop runs toward a transponder suspended at various depths below a small boat.

- Benthos Order Status:

  - All mechanical parts are in-house and production is expecting an on-time delivery in two weeks.

  - Changed the order of the base-station modem to include the software necessary to access both serial ports.  I was told the new hardware had two available serial ports but eventually it was pointed out that in order to use both ports I was supposed to have purchased that $1800 upgrade.

  - Benthos confirmed our proposed mounting configuration inside the hull is compatible with their on-board compass – which we aren’t using…

  - We did a bench-top test to confirm the magnetic signature of the modem (during transmit),  is below the noise threshold of our primary navigation compass and the proposed separation distance.

  - Two test modems were provided to Wayne for bench-top testing

- AUV

  - Long nose cone is on track for delivery from Waterat in two weeks

  - Brian has looked at the DAT interface and will start developing the driver next week

  - Rob has begun porting the QNX docking code to the Tethys code-base

  - We modified the front endcap of Daphne to support the 12 pin connector required for the DAT.  We plan to modify Tethys next week

  - We modified the front endcap wiring harness documentation to support the DAT.  A request for ETech has been submitted to make the change

  - Robert is working on the brackets to hold the PCBs and Transducer

  - New long-nose sensor brackets are in the shop for fab