Niobium connector for AUV recharge. Preliminary experiments

François Cazenave

Date: 2/19/2020

Step 1: Test in small seawater tank.

Measure resistance of contact between two niobium rods in seawater.

- Experiment with a range of voltages and currents.

- Experiment with a range of contact forces and contact areas.

- Experiment with sliding contact

- Experiment with rod that has been submerged in seawater for a long time.

- Experiment with other metal touching or scrapping Nb rod: does it cause short to seawater

- Experiment with data over power

Seawater return:

- Experiment with seawater return in tank, including data over power.

- Consider corrosion implications (consult Jenkins if needed)

Step 2: Test in MBARI test tank (assuming seawater return works)

Experiment with basic latching in MBARI test tank

- Fabricate latching wire using 0.25” tubing, hang in tank.

- Integrate Nb contact in blue ROV with gripper

- Experiment with latching in the test tank.

Step 2: (assuming seawater return does not work)

Design 2-contact connection mechanism.