| Project Number: | 902106 |
| --- | --- |

| Project Start: Jan. 1, | 2027 | Project Duration (in years): | 1 or 5 |
| --- | --- | --- | --- |

AUV Docking Development and Science Operations

Project Manager:		      François Cazenave

Principal Investigator:		      Brett Hobson, Andy Hamilton

Lead Engineer:			      François Cazenave

Note: Not all projects  require entries for each  title. Please only indicate individuals for titles appropriate for your project. Definitions for each position are found in the instructions for internal 2027 proposals.

- PROPOSAL (all projects)

Is this a feasibility study [<100 days (800 hours) of total labor requested]?

Yes

x    No

Abstract or Background

Please provide a copy of the project abstract or background information. Please be sure to address: What fundamental problem does this project address? What makes this work relevant to MBARI (see our Strategic Roadmap Priorities and Challenges) and the Packard Foundation? Where does this work fit into the ~5–10 year plan for your group, at MBARI, and in the field?

Abstract/background narrative:

Over the past several years, MBARI has developed a Niobicon-based AUV docking system that has been successfully deployed in Monterey Bay to support extended science missions. Two complete docking moorings have been built—one powered by solar panels and the other by a wave-energy buoy—and three LRAUVs have been equipped with docking hardware.

During the next five-year period, we will continue local deployments while expanding docking capabilities to additional vehicles and payloads. These deployments will serve as a testbed for refining docking procedures and advancing system autonomy, reliability, and operational endurance.

In 2027, we plan to make several upgrades to the system to improve its reliability and autonomy.

We are also planning to bring AUV docking to greater depths by integrating the docking hardware in the Deep LRAUV. Starting in May, we will deploy the wave-powered dock and perform engineering tests for one month. Subsequently, we plan to perform a month-long survey, with the Deep LRAUV and planktivore, to quantify variability in the particle field above the Sinker instrument at MARS (in collaboration with Monique Messié). In parallel, we are planning to design and build a docking station to be deployed at MARS in early 2028. It will stay there for one year and will support surveys in that area.

Beyond 2027, we will pursue several extensions of the technology in collaboration with MBARI scientists to enable new scientific applications:

We will investigate a battery-powered seafloor docking station capable of supporting 5–10 LRAUV recharges without a surface buoy. Free-fall deployment would enable operations in remote locations and at greater depths than current surface moorings. This effort builds directly on existing docking technology while requiring further advances in automation and reliability.

We are also interested in using the docking technology to autonomously transfer energy and data between a surface energy source and remote seafloor instruments. Such a system could significantly extend the deployment duration of power-intensive instrumentation.

This work will provide unique capabilities for scientific investigations that cannot currently be achieved through other approaches. As the docking technology remains unique to MBARI, it positions the institution to lead the development of novel autonomous marine systems. We will also continue collaborating with Saab and Northrop Grumman to transition and deploy this technology where appropriate.

Select relevant challenges in the table below, and complete the narrative in the space provided above.

| Strategic Priorities and Challenges |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Innovate and Build |  | Explore and Protect |  | Inspire and Engage |  |
| Build scalable technologies to improve ocean health assessments | x | Explore and observe life in a dynamic ocean |  | Share what we learn to inspire and inform ocean conservation |  |
| Map the deep seafloor at high resolution |  | Study the impacts of climate change and other ocean threats |  | Make ocean science and engineering careers accessible to all |  |
| Advance ocean-based artificial intelligence |  |  |  | Invest in our people and foster a diverse and inclusive workforce |  |

Does this activity relate to one or more 2027 proposals your team is submitting?

Is this effort linked to other internal or externally funded projects or collaborative efforts? If you plan to submit more than one Phase 1 proposal (internally or externally funded), please provide a list of those projects by Project or DQ#, and please make clear how this project relates to the others you plan to submit and where it ranks in your overall priorities. Does this proposal take precedent over other ongoing or planned activities? Please complete the tables as indicated, and provide any additional text in the space below. (Add more rows as needed.)

| Project # | Project Name | Start Year | End Year | Priority (1, 2, 3, etc.) |
| --- | --- | --- | --- | --- |
| 902101 | LRAUV Technology Development | 2027 | 2031 | 1 |
| 901405 | Wave-Energy Conversion for Oceanographic Applications | 2026 | 2028 | 1 |
| 800175 | NOWRDC | 2025 | 2027 | 1 |
| 705102 | Open Wave-Energy Control System Development Platform | 2021 | 2028 | 1 |
| 708190 | BIL Activities at PMEC | 2025 | 2029 | 1 |

Add additional details here, as needed:

The proposed continuation of AUV docking technology development is closely related to two internal projects, LRAUV Technology Development and Wave-Energy Conversion for Oceanographic Applications. There is considerable overlap in the teams and the AUV docking efforts rely on both projects; needed LRAUV developments are provided by the LRAUV team, and the wave-energy converter deployments provide the needed energy for Monterey Bay testing and use of the docking platform. Several externally funded projects are also listed above as requested, but with the exception of NOWRDC, these do not have a direct dependence on AUV docking.

Continuing Projects Only: Status Report

Provide a brief update of the current project relative to what was proposed for 2026. If the project includes multiple subprojects(or milestones), please list each under a subheading as you deem appropriate.

Current status of project to date, including results and any significant changes from the previous project proposal

We have spent the first half of 2026 focusing on preparing the two docking stations and two docking LRAUVs needed for the NOWRDC demonstration planned for this fall. We have made significant progress and we expect to meet our goals. We have made numerous upgrades that have been time consuming. The solar docking station will be deployed in early September and the wave-energy docking station shortly after. As planned, we will work on video-guided homing on the dock later in the year.

What percentages of labor and budget have been used through the 1st half of the year?

So far we’ve used about 50% of labor and budget in 2026

Will you complete the current year’s scheduled work?

Yes, we are planning to complete the scheduled work.

List the last three peer-reviewed publications and/or other significant products, data sets, education and/or outreach activities, and similar activities that have resulted directly from this project? Please be brief.

Hamilton presented this work as an invited speaker at the UC Berkeley ME/Ocean Engineering Seminar Series in May 2026.

All Projects: Major Activities for 2027

Research and/or Engineering Plans

Please provide an outline of research activities and/or engineering development plans (WBS, schedule, design, design reviews, manufacturing, prototyping, I&T, test deployments). Milestones—significant events in the timeline of a project useful for tracking progress—should be referenced here and included in the milestone chart below, as appropriate for your project. Include a maximum of five milestones.

For 2027, the project plans to concentrate on three areas. First we will be re-deploying the wave-energy dock to support LRAUV operations and science (the solar dock will be kept as a backup). In 2026 we developed considerable new hardware that will get fairly limited use in 2026 as part of NOWRDC.  More deployment time is needed to refine these systems and operations, and we have some internal science use-cases that will take advantage of the deployment in 2027.  Second, we are planning to do the engineering development required to create an AUV docking station at MARS, with an aim to deploy in early 2028.  Although much of our current designs can be re-used for this application, the installation costs and potential for a year-long installation dictate more engineering is performed to ensure a reliable solution.  Finally, we will continue to work to find appropriate use-cases for our other ambitions for this capability, including battery supported AUV docks in remote locations, a collaborative Axial seamount installation, bi-directional power-transfer, and improved reliability and autonomy.

Engineering development plans

- Maintenance, upgrades and engineering tests

From the solar panels to the power-transfer device, the docking system includes numerous complex sub-systems that require maintenance, upgrades and testing. We plan to spend much of 2027 doing this important work. Below is a list of the tasks to be performed:

1.1 Solar power docking station:

- Install the newer Ethernet-over-power modems (replace modems, re-terminate the stretch hose, replace the seafloor cable).

- Improve the energy efficiency by removing unnecessary loads, generate a detailed energy budget.

  - Investigate  adding another energy source, such as a diesel generator, a fuel cell or a wind turbine (this will be necessary in cases where the wave-energy buoy cannot be used and the energy demand is high).

- Implement an auto-start feature, allowing the LRAUV to “wake-up” the docking station and to dock without human intervention  (this also applies to the wave-energy dock). This requires hardware changes in the subsea hardware.

- Perform a complete overhaul of the docking buoy. After many months at sea, this has become necessary.

1.2 Wave-energy docking station:

- In the fall of 2026, we will be deploying the wave-energy docking station for the first time. Based on the performance of the system, we will make necessary repairs, changes or upgrades.

- We will upgrade the mooring cable terminations to allow deployment in year-round conditions, and for longer periods.

1.3 Power-transfer device

The power-transfer device includes the niobium dock, the latch and electrical contacts on the LRAUV, and associated electrical components. While this system is fairly mature and well tested, several tasks are needed:

- Increase the charging voltage from 24 VDC to 48 VDC. This will  double the power transfer efficiency and the charging speed.

- Replace the diode bridge with a higher efficiency unit.

- Improve the documentation

- Currently, electrical power can only flow from the dock to the AUV. We plan to investigate 2-way power transfer, which would allow the use of the LRAUV to deliver energy to a remote location. This feature will be tested in the lab only in 2027.

1.4 AUV homing

- Homing is the process of the AUV acoustically locating the dock, flying towards it and making contact with it. Improving the success rate is important to save time and energy. The LRAUV teams will continue testing and making refinements to the procedure.

- We are planning to start testing video-guided homing in the test tank and on the ‘mock dock’ (a very simple version of the dock used for testing) in late 2026. Depending on the result of this first study, we will continue and start testing at sea on the real dock. This would require installing homing lights on the dock in 2027.

- Improve the autonomy of the system to demonstrate the possibility of an autonomous seafloor battery-powered docking station, where human intervention would not be possible.

1.5 Mission planning

Planning long LRAUV missions supported by the dock is more complicated than standard missions on a single charge. Available energy, time spent charging, payload energy use and speed and distance travelled all factor in. We plan to develop a software tool to help scientists plan LRAUV missions supported by the dock.

2. Taking docking to greater depths and MARS

We have long been interested in installing a docking station at MARS, which presents ideal conditions for a dock: ample electrical power and fast, reliable, communications. Moreover, biofouling and wave-induced wear will be greatly reduced compared to shallow deployments.

Docking at MARS greatly expands the reach of the LRAUV. Scientists have expressed interest in working in that area (Durkin, Messié, Haddock, Ryan).

We plan to rely on the 1500-m rated LRAUV, Opah. We will adapt the latch and the power-transfer device to it, while the dock itself will stay essentially the same. The cabling, connectors and power electronics between MARS and the dock will be similar to those used on other MBARI instruments deployed there.

Our goal is to have the system fully designed, built and tested in the lab by the end of 2027 and to deploy it at MARS in early 2028.

Engineering tasks for integrating the docking hardware to LRAUV Opah:

- Produce solidworks drawing of the power-transfer device on LRAUV (current components are already rated to 1500 m depth and will be compatible with Opah) (January-February).

- Order components and make necessary modifications to Opah (March).

- Assemble device on Opah and perform docking tests in the test tank (April)

- Test docking at sea on the dock (May).

- Start using it for science (June).

Engineering tasks for MARS docking station development

- List the requirements of the system

- Produce high level schematics, mechanical drawings and deployment plans

- Preliminary design review (April)

- Produce complete set of drawing

- Critical design review (July)

- Order components and start assembly

- Test in the lab and the test tank (November, december)

- Deploy at MARS in early 2028.

Field operations and science:

Field operations for 2027 are listed below:

- We will deploy the wave-energy dock in early May, most likely at the usual wave-energy buoy site.

- we will perform engineering tests for one month

  - Test new features

  - Test docking with Opah and Planktivore

  - Test video-guided homing

- In June we will run a month-long survey for Monique Messié. The goal of the survey is to characterize sinking particles above the SINKER instrument plugged at MARS throughout the water column (surface-900m) using Opah and Planktivore. Because temporal variability is expected to be high (based on SINKER data), getting a good representation of typical conditions during the upwelling season will require being able to observe these particles for longer than a few days (i.e. if using Opah without docking). We would operate Opah for one month, recharging at the dock as needed. The survey could be bracketed by Steve Haddock's midwater requests on the way out and back. The MARS Opah survey would be very simple (such as a profile_station or bow-tie pattern) and repeated for days at a time, so that piloting is easier than during other LRAUV missions (including bioluminescence docking surveys).

- Later in the summer and the fall, we will continue engineering tests and science missions on an ad hoc basis.

Specific Deliverables

Please briefly describe the anticipated deliverables (e.g., applications / tools, datasets, products) for the project.

1- Docking hardware for Deep LRAUV built, installed on Opah, and tested in the lab.

2- Solar buoy upgraded: new communications device installed, battery in subsea housing installed, efficiency and power generation improved.

3- Wave-energy mooring serviced, upgraded and prepared for deployment.

4- Wave-energy dock deployed, engineering tests and science mission performed at sea.

5- MARS docking station preliminary and design critical design review performed..

6- MARS docking station components ordered and fabricated.

Field programs (detail ship requirements/other asset deployments/diver requirements)

In addition to describing anticipated field programs, please explain clearly all requirements for shared assets,  platforms, and equipment (Dorado AUVs, LRAUVs, Wave Glider, MiniROV, etc.) for each proposed field program.

R/V Shana Rae will be contracted for a total of 4 days to deploy and recover the docking moorings.

R/V Paragon may be used for occasional short trips for buoy maintenance.

There will be one deployment of the docking station, coupled with the wave-energy buoy, from May to December 2027. The dock will be used to support:

- LRAUV Pontus, for several engineering tests

- LRAUV Opah, for engineering tests and science from May to July 2027

- science missions, on ad-hoc basis

The solar docking station may be deployed instead of the wave-energy one, depending on the state of the 2 systems and the needs of the science mission.

AUV operations during these experiments will be performed as part of 902101 and other projects. R/V Paragon may be needed for occasional maintenance on the mooring.

In 2028, we plan to request 1 or 2 days of ROV Ventana to deploy the dock at MARS. We expect maintenance will be needed yearly, requiring 1 or 2 days of ROV Ventana each year.

Asset resource requests must also be entered in the budget entry system. Note that AUV deployment numbers and diver requirement questions are also included in the Compliance and Planning Considerations Form described at the end of this proposal form.

AUV days will be requested as part of LRAUV lab or Science users

Milestones and required personnel (see personnel classification list for Engineering and DMO  staff classifications)

Please complete the milestone chart as necessary for your project. The milestone chart must clearly account for all of the shared labor resources required, grouped by project milestones. All labor resources listed in the milestone chart must be entered into the Adaptive accounting system. You may use initials for specific individuals.

Please verify that the labor entries in Adaptive match the entries in the milestone chart BEFORE submitting the final proposal.

| 2027 Project Start Date: 01/01/2027 | Milestone 1 | Milestone 2 | Milestone 3 | Milestone 4 | Milestone 5 | Milestone 6 | Total Resources (in hours) for each resource category for all milestones listed here |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Milestone Date | 04/01/2027 | 05/01/2027 | 05/01/2027 | 07/01/2026 | 09/01/2025 | 11/01/2026 |  |
| Milestone Description | Deep LRAUV hardware built and installed on Opah | Solar buoy upgraded | Wave-energy mooring prepared for deployment | Engineering tests and science mission performed at sea | MARS docking station preliminary and critical design review | MARS docking station components ordered and fabricated |  |
| Milestone priority | 1 | 6 | 2 | 3 | 4 | 5 |  |
| Electrical Engineer | Jensen 20 | Jensen 40 | Jensen 80 | Jensen 40 | Jensen 180 | Jensen 40 | 400 |
| Software Eng-Embedded | Kieft 10 | Kieft 10 | Kieft 10 Ander. 60 | Kieft 10 Ander. 60 | Kieft 10 |  | 50+120=170 |
| Mechanical Engineer | Ham. 50 Caz. 100 Hob. 20 | Ham. 50 Caz. 100 Hob. 20 | Ham. 50 Caz. 100 Hob. 20 | Ham. 50 Caz. 100 Hob. 20 | Ham. 50 Caz. 100 Hob. 20 | Ham. 50 Caz. 100 Hob. 20 | 300 +600+ 120 = 1020 |
| Electrical Tech | 40 | 40 | 40 | 40 | 40 | 40 | 240 |
| Mechanical Tech | 80 | 80 | 100 | 100 | 0 | 100 | 460 |
| EE Tech | McClure 60 | McClure 40 | McClure 80 | McClure 40 | McClure 60 | McClure 80 | 360 |
| Machinist | 100 | 40 | 40 | 40 | 0 | 200 | 420 |

*E.g., enter a rating of 1 for highest priority, 5 for lowest priority.

Budget justification

Enter budgets in the Adaptive accounting system and provide descriptions and justification for each request in the system. Label each expense description in Adaptive with an identifier (e.g., Milestone 1: Materials for EM Cable Terminations $4,000). If details in Adaptive are tied to specific tasks, then no further justification is required. If budget requests are not explained in Adaptive, please add more detail here.

The totals for each budget request and category detailed here must match the totals entered in Adaptive. Please verify BEFORE submitting the final proposal.

We are requesting $103200 for hardware, materials, equipment and supplies. (This budget could be reduced by reusing components from the existing docking stations but it would add some risk).

- Material and components to upgrade the existing docking moorings: $8000

  - $2000 for ethernet over power modems and other electronics for solar dock

  - $2000 for miscellaneous supplies

  - $4000 for new cable terminations for wave-energy mooring cable

- Materials and components for the fabrication of the docking hardware to be installed on Deep LRAUV Opah: $17700

  - $6700 for 1 Ultra Motion actuator (for docking arm)

  - $800 for docking payload housing and electronics

  - $1200 for connectors

  - $1000 for printed parts

  - $500 for hardware

  - $1000 for two sets of niobium brushes

  - $6000 for one LRAUV nose  shell

  - $500 for miscellaneous materials.

- Materials and components for the fabrication of the new MARS dock, associated electronics and devices: $63500

  - $3000 for 1 docking rod

  - $5000 for material for 2 titanium housings (MARS interface housing and main dock electronics housings)

  - $2000 for syntactic float

  - $2000 lines, cables, hardware, anchor

  - $12000 for Benthos acoustic modem and transducer

  - $2100 for DSPL Nano Sealite (dock light)

  - $9400 for DSPL HD multi SeaCam (dock camera)

  - $2000 for Birns connectors

  - $4000 for titanium Subconn connectors

  - $2000 camera wiper

  - $10000 for ODI connector

  - $4000 for dock electronics (modem, power supplies, circuit boards, etc)

- Miscellaneous expenses: $5000.

- 4 Days on R/V Shana Rae to deploy and recover the solar dock: $9000.

Project team

Specify internal and external collaborators.

The team in 2027 will be similar to 2026’s.

François Cazenave: project management, mechanical design, lab testing, fabrication, assembly and deployment supervision.

James McClure: electrical design and assembly.

Scott Jensen: electrical design.

Andy Hamilton: system engineering, project management, IP issues.

Brett Hobson: mechanical design, electrical design, LRAUV integration and operations. Much of Brett’s labor will be requested as part of 902101.

Brian Kieft: Docking related software on LRAUV, in-lab and at-sea testing. Much of Brian’s labor on the project will be requested as part of 902101.

Quinn Shemet: Docking related software on LRAUV, in-lab and at-sea testing. Much of Quinn’s labor on the project will be requested as part of 902101.

Michael Anderson:  Docking related software on LRAUV and video guidance systems.

External consultants/contractors

If you propose to engage an external consultant/contractor, please attach a sufficiently detailed SOW and budget proposal from the vendor. If approved, a fully executed Independent Contractor Agreement must be in place before work can begin.

Long-Term Planning Considerations

Please describe any commitments on internally or externally funded projects that have implications beyond 2027. This could include commitments of shared assets, labor resources, ship time, international multiyear programs, large-scale expeditions, etc.

The work we will do in 2028 to 2031 will depend on the results of the work of previous years, therefore, we cannot make detailed plans. But we have several ideas we would like to explore and we will work with MBARI scientists to determine specific applications of interest. In particular we will be looking for applications that provide a science result that cannot be obtained in any other manner.

Seafloor Battery-Powered AUV Dock: While the energy harvesting buoys provide a long-term energy supply and can support a number of vehicles simultaneously, the required buoy systems are complicated and expensive to deploy, and may not be required in all applications. Therefore, we are interested in pursuing a coupling of the AUV docking technology to a smaller stored-energy solution (batteries) that do not require a surface buoy and would allow 5-10 LRAUV recharges. Such a system would involve a very simple and fast free-fall deployment, followed by an LRAUV deployment that would then be supported by the dock located on the seafloor. The simplicity of deployment would enable these capabilities to be performed in remote locations and enable science data collection over extended times and seasons that would otherwise be impossible.

This development is a straightforward extension of the work that has been done so far, but would require further work to be done on the docking automation and reliability elements. This testing of such behaviors can be done locally using the buoy-supported systems in Monterey Bay.

Cabled Seafloor Observatories: We have long been interested in developing an LRAUV docking installation at Axial Seamount. We have participated in various workshops discussing a resident vehicle that would be supported by the Ocean Observing Initiate power and data node in the pacific northwest. Additionally, we have worked in the past on pursuing funding for such a deployment and plan to continue to explore this in the upcoming project period. Our immediate plans to develop a docking station for MARS to support our MBARI scientists is an important step towards supporting other cabled seafloor observatories.

Power Delivery AUV: The Niobicon docking technology has the capability for bi-directional power-flow in addition to the AUV charge-only scenario currently implemented. Development of this capability would enable using an AUVs as a power-delivery vehicle, shuttling power and data between an energy source (e.g. the wave-energy buoy) and instrumentation on the seafloor. We envision being able to support energy-intensive instrumentation without requiring a physical connection. The Distributed Acoustic Sensing system  (DAS) is a good example in which the deployments involve a complicated cable-lay process that would be challenging to connect directly to an existing seafloor located device. For this reason, a power-delivery system may be the best way to extend the deployment durations of this type of energy-intensive instrumentation.

Compliance and Planning Considerations

All responses must be recorded in the Google form linked below. A form must be submitted for every project.

Link to Google Compliance and Planning Considerations form  -OR- https://forms.gle/HW2tJDJdPzXHadyg9