BlueBoat VN300 Serial Logger Instructions

Access Information:

- Host/SSH: 192.168.2.2

- Username: pi

- Password: raspberry

Logging Procedure:

- Connect to the device via SSH using the provided credentials.

- Once connected, execute the following command to start the VN300 data display and initiate recording using nohup: nohup python3 serial_logger.py

  - This command runs the Python script in the background (nohup ignores hangup signals).

  - Connect to FileZilla (see instructions below) and refresh to see the IMU.csv and GPS.csv file sizes incrementing while logging

  - Use Contol+C To stop logging

- Confirmation Message:

  - Upon successful execution, you should see the message: nohup: ignoring input and appending output to ‘nohup.out’

    - This indicates that the logger is actively recording data.

Handling Terminated SSH Sessions:

- If the SSH session terminates unexpectedly, the logging process will continue.

- To manually stop logging after re-establishing an SSH connection:

  - List all processes running Python scripts: ps aux | grep python3

  - Identify the process ID (PID) associated with serial_logger.py.

  - Terminate the process using: kill PID

Using screen to View Serial Data:

- To monitor serial data in real-time:

  - Open a screen session: screen /dev/ttyUSB0 115200

  - To detach from screen without terminating the session:

    - Press Ctrl + A, then Ctrl + D.

  - To terminate the screen session:

    - Press Ctrl + A, then Ctrl + \ and confirm with y.

Downloading Data via FileZilla:

- Use FileZilla or a similar FTP client to download logged data:

  - Connect to the device using the same SSH credentials (Host, Username, Password).

  - Navigate to the directory where serial_logger.py is saving data.

  - Transfer the desired files to your local machine for analysis.

Restarting the Raspberry Pi:

- To restart the Raspberry Pi using SSH:

  - Connect via SSH and execute: sudo reboot

  - Confirm the restart process.

Note: Ensure proper network connectivity and permissions for seamless operation of the logging and data retrieval processes.

### GPS Time Synchronization Script Instruction Manual

#### Overview

This script reads GPS data from a serial port on your Raspberry Pi and synchronizes the system time to UTC using the $GPRMC NMEA sentence. It logs the synchronization process, including old and new system times and any errors encountered, into a CSV file named gps_time_sync_log.csv.

#### Requirements

- Raspberry Pi with Python installed (Python 3.x recommended)

- GPS module connected via serial port (adjust SERIAL_PORT and BAUD_RATE variables if necessary)

- sudo access for setting system time using date -u

#### Setup Instructions

- Install Required Libraries: Ensure Python and necessary libraries (serial, csv) are installed.

bash

Copy code

pip install pyserial

- Set Raspberry Pi Timezone to UTC: Use raspi-config to set the Raspberry Pi's timezone to UTC.

bash

Copy code

sudo raspi-config

  - Navigate to Localization Options -> Change Timezone -> None of the above -> UTC.

- Download and Configure the Script:

  - Download the script (gps_time_sync.py) to your Raspberry Pi.

  - Edit the script:

    - Adjust SERIAL_PORT and BAUD_RATE variables to match your GPS module specifications if needed.

- Run the Script:

bash

Copy code

python3 gps_time_sync.py

  - The script will open the specified serial port (SERIAL_PORT), read GPS data, parse $GPRMC sentences, and synchronize the system time to UTC.

  - It logs the synchronization process, including old and new system times and any errors encountered, into gps_time_sync_log.csv.

- Review Logs:

  - After running the script, check gps_time_sync_log.csv for detailed logs of each synchronization attempt.

  - Logs include timestamps, old system time, new system time after synchronization, and any errors encountered.

#### Troubleshooting

- No GPS Data Received:

  - Ensure the GPS module is properly connected and powered.

  - Verify the serial port (SERIAL_PORT) and baud rate (BAUD_RATE) settings match your GPS module.

- Time Synchronization Issues:

  - Check gps_time_sync_log.csv for error messages and review Raspberry Pi's connectivity and GPS reception.

#### Notes

- Ensure the script has necessary permissions (sudo) to set system time using date -u.

- Customize error handling and additional functionalities as per specific requirements.