- EAHRS_TYPE = 1 (VectorNav)

- EAHRS_SENSORS = 15 (all)

- EAHRS_RATE = 50 (set this up in control center software)

- GPS_TYPE = 21 (ExternalAHRS)

- GPS_TYPE2 = 21 (ExternalAHRS)

- SerialX_Protocol = 36 (AHRS) – Whichever serial port the VN-300 is plugged into, change this parameter

- SerialX_Baud = 115 (Confirm Baud of VN-300)

- AHRS_EKF_TYPE = 11 (External AHRS)

- SERIAL3_Options -> Used as null modem (if needed)

INS_USE

Others:

- WP_SPEED – this is the speed of the vessel in auto mode.

More info here