|  | Simulation of testAssemblyNewHousing Date: Tuesday, August 29, 2023 Designer: kiwi Study name: Static 4 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 5 Connector Definitions 5 Interaction Information 6 Mesh information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 9. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: testAssemblyNewHousing Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Combine10; Solid Body; Mass:1.88206 kg Volume:0.000697059 m^3 Density:2,700 kg/m^3 Weight:18.4442 N; C:\Users\kiwi\Documents\902204_CoMPAS\ME.Drawings\Mola Camera Short Version\AMoleCamHousingV2.SLDPRT Aug 28 14:16:57 2023. Combine4; Solid Body; Mass:1.18625 kg Volume:0.000422152 m^3 Density:2,810 kg/m^3 Weight:11.6252 N; C:\Users\kiwi\Documents\902204_CoMPAS\ME.Drawings\Mola Camera Short Version\Lid2.SLDPRT Aug 28 15:26:46 2023. Fillet6; Solid Body; Mass:0.136197 kg Volume:0.000113497 m^3 Density:1,200 kg/m^3 Weight:1.33473 N; C:\Users\kiwi\Documents\902204_CoMPAS\ME.Drawings\Mola Camera Short Version\windowSized.SLDPRT Aug 29 08:50:15 2023.

# Study Properties Study name; Static 4. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\kiwi\Documents\902204_CoMPAS\ME.Drawings\Mola Camera Short Version).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 2.75e+08 N/m^2. Tensile strength:; 3.1e+08 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.6e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 1(Combine10)(AMoleCamHousingV2-1). Curve Data:N/A. Name:; 7075-T6 (SN). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 5.05e+08 N/m^2. Tensile strength:; 5.7e+08 N/m^2. Elastic modulus:; 7.2e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,810 kg/m^3. Shear modulus:; 2.69e+10 N/m^2. Thermal expansion coefficient:; 2.36e-05 /Kelvin.; SolidBody 1(Combine4)(Lid2-1). Curve Data:N/A. Name:; Acrylic (Medium-high impact). Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 4.5e+07 N/m^2. Tensile strength:; 7.3e+07 N/m^2. Elastic modulus:; 3e+09 N/m^2. Poisson's ratio:; 0.35. Mass density:; 1,200 kg/m^3. Shear modulus:; 8.9e+08 N/m^2. Thermal expansion coefficient:; 5.2e-05 /Kelvin.; SolidBody 1(Fillet6)(windowSized-1). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -552.312; -346,490; 3.9366; 346,491. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Pressure-1; Entities:; 21 face(s). Type:; Normal to selected face. Value:; 2,250. Units:; psi. Phase Angle:; 0. Units:; deg..

# Connector Definitions No Data

# Interaction Information Interaction; Interaction Image; Interaction Properties. Global Interaction; Type:; Bonded. Components:; 1 component(s). Options:; Independent mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 0.844462 in. Minimum element size; 0.0422231 in. Mesh Quality; High. Remesh failed parts independently; Off. ## Mesh information - Details Total Nodes; 464523. Total Elements; 304668. Maximum Aspect Ratio; 891.99. % of elements with Aspect Ratio < 3; 97.4. Percentage of elements with Aspect Ratio > 10; 0.24. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:24. Computer name:; GREATWHITE.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -552.312; -346,490; 3.9366; 346,491. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 6.92848; -31.2327; -27.374; 42.1049. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 2.246e+05N/m^2 Node: 463039; 5.877e+09N/m^2 Node: 167145. testAssemblyNewHousing-Static 4-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 303785; 3.073e+01mm Node: 457920. testAssemblyNewHousing-Static 4-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.862e-06 Element: 66752; 4.767e-01 Element: 280753. testAssemblyNewHousing-Static 4-Strain-Strain1.

# Conclusion