MBARI Project Number: 902305

 Link to load this expedition into the database:
 https://mww.mbari.org/expd/log/precruise.asp?step=1

 (Copy from the fields below into the web form fields)

 ExpeditionID:
 ShipName: rcsn
 ShipSeqNum:

0600
 Purpose: Recover instruments

Equipment to be recovered:
SES (ROV pull pin release of autonomous lander)
McLane Sediment trap aka “Trigger trap” (ROV pull pin release of autonomous lander).
2 current meters (ROV direct recovery and bring up in drawers. These are the ones that look like baseball bats)

Order of operations:
1. Steam to McLane Sediment trap (36 42.7506 N 122 11.1658 W—this is at-bottom lat/long. It drifted some from launch point)

2. Launch ROV, find instrument (Homer 26, 880 m depth)

3. Inspect briefly.

4. Recover current meter #1 (very close to McLane trap, also at 880 m depth).

5. Release float line and pull pin on McLane Trap.

6. Transit to SES (36 42.6554 N 122 10.8600 W, Homer 10 on south side of instrument).

7. Inspect SES.

8. Recover Current meter #2 (Very close to SES, 844 m depth).

9. Release float line and pull pin on SES.

10. Recover ROV

11. Find and recover SES: VHF: 160.725; Argos 401.65400; Homer 10

12. Find and recover McLane trap: VHF: 154.585; Argos 401.64800; Homer 26

13. If time, re-launch and recover one or both weight plates

Equipment Requested: ROV
 Equipment Details: Benthic sled for carrying current meters back.
 Winch Requested: None
 Winch Details: None
 Mooring Deployment Details: SUBSEA_MOORING_DEPLOYMENT Not a mooring,
 but is two large landers: SES, and McLane trap

Participants:
MBARI Personnel:

Crissy Huffard
Rich Henthorn
Irene Hu

Paul McGill
Colleen Durkin
Evan Mattiasen

RegionDesc:
 PlannedTrackDesc:
 MARS, but not plugging in
 Track Outside Monterey Bay: NO
 Track In Restricted Areas: NO
 MBNMS Permit Compliance: YES
 CDFW Permit Compliance: NOT_APPLICABLE SMPA Collection Permit
 Compliance: YES