MBARI Project Number: 902305

ShipName: rcsn

Purpose:

Equipment to be deployed:
SES (as autonomous lander)
McLane Sediment trap (as autonomous lander, aka Trigger trap/TT).
One current meter (looks like a baseball bat, which we deployed last year)

Order of operations (let me know if this should be altered):
1. Steam to MARS. We aim to drop the SES and TT ~400 m apart. Exact lat and long doesn't matter so long as we're not dropping our equipment on other people's instruments or on the slope. The last SES deployment location was 36 42.6554 N 122 10.8600 W

2. Deploy TT as lander
3. Steam to SES drop location, deploy SES
4. ROV dive:

- Descend to SES. Fly around to see how it looks, fly over and look down to see if anything is on top of the baffle.

- Position the SES so that modem is on the side facing the TT.

- Deploy 1 current meter at SES.

- Get video of SES for SciComm

- Transit underwater to TT.

- Fly around TT to see how it looks, fly over and look down to see if anything is on top of the baffle.

- Position the TT so that modem is on the side facing the SES.

- Get video of TT for SciComm

- Recover ROV

- Not sure if Rich and Paul will want to talk with the SES and McLane traps modem via a dunker. Probably not, but just putting that here just in case.

ScheduledStartDtg: 2024-12-06 0600 Local Moss Landing time
ScheduledEndDtg: 2024-12-06 1630 Local Moss Landing time
EquipmentDesc:
Equipment Requested: ROV
Equipment Details: Benthic sled for carrying current meter down.
No collections of any kind anticipated.
Hazmat: Deploying sediment trap with cups that have a total of <3L of <10% formalin.

96 lithium primary DD cells in the SES battery, completely contained within the titanium sphere
Winch Requested: None
Winch Details: None
Mooring Deployment Details: SUBSEA_MOORING_DEPLOYMENT
 Not a mooring, but is two large landers: SES, and McLane trap
RegionDesc:
PlannedTrackDesc:
MARS, but not plugging in
Track Outside Monterey Bay: NO
Track In Restricted Areas: NO
MBNMS Permit Compliance: YES
CDFW Permit Compliance: NOT_APPLICABLE
SMPA Collection Permit Compliance: YES

Personnel:

Crissy Huffard
Rich Henthorn
Irene Hu
Paul McGill
Colleen Durkin
Ben Jansen

SciComm team possibly sending 2 people