| Item | Description | Person | Timeline goal |
| --- | --- | --- | --- |
| Deployment prep |  |  |  |
| Hardware: Prep McLane trap SN ML-12813-01 | Add floats, figure out ballast, add baffle, add modem, add batteries to controller, Squirty | Evan, Crissy, Paul | June 1 |
| Hardware: Squirty | How often does it squirt? 2 x a day. We might want to go for 4 x a day if we can | Paul | May 15 |
| Hardware: Squirty | We have batteries. Ordered from PBS | Done | Done |
| Software: Triggering | Use 80% of max slope, from 1153 rolling mean | Rich, Crissy | Decided |
| Software: Collect schedule | Once it’s triggered, how long does it stay open for? Compare SES MARS flux estimates to Station M fluxes, make sure doesn’t clog. Aim to fill cup ¼ or less. Avoid clog danger zone. Based on Pulse 70 cups 6,9: < 30 mg C/ m^2/ d x 17 d collect time at Station M = 510 mg C/m^2. Estimating MARS flux could be as high as 1000 mg C/m^2/. So- let’s go for a 12 h collect time to be extra sure. | Crissy | ASAP |
| Software: Collect schedule | Once it has triggered, don’t add a wait period for the next trigger |  |  |
| Software: Collect schedule | Once it triggers, goes for how many cups? 1 cup, 12 hours each. | Crissy | ASAP |
| Software: Prep McLane trap | Configure trap schedule Configure interface with modem | Rich, Crissy, Paul | June 1 |
| SES prep | Configure trigger, Configure control triggering, Add weights and baffle, New batteries | Rich, Crissy, Paul | 1 June |
| Current meters | Change batteries, prep | Crissy | 15 May |
| How far from each other should/can they be deployed? | How far away would we deploy moorings from each other | Start with 1 km to make sure there’s enough time for transit. Deploy both then transit along the seafloor if they can go ½ a knot | Crissy and DJ Deploy McLane trap, Deploy SES, Dive on SES, transit to McLane trap and check that out. |
| Current meters- 2Dacmplus and stand | Stand | Only if easy | Paul to do, but only if easy |
| SES battery | 3 x an hour might get us 2.5 months | Paul | By next week |
| pH Sensor |  | Crissy |  |
| Deck test | Put both in test tank area on power, mission script And run tests of triggering TAKE OFF WIPER Add a piece of black paper on the slide to make sure it adds to data A brief in water test after bench tests | All | 15 May bench 1 June in water |
| Data analysis |  |  |  |
| Trigger algorithm simulation testing | Quantify false positives and negatives with different scenarios: SES data from MARS, SES data from Station M, Station M POC flux data | Crissy | After deployment |
| Modem | test |  | Get deck box and try to talk to it. |