| Item | Description | Person | Timeline goal |
| --- | --- | --- | --- |
| Deployment prep |  |  |  |
| Test tank 6 June for testing comms |  |  |  |
| Hardware: Prep McLane trap SN ML-12813-01 | Add floats, figure out ballast, add baffle, add modem, add batteries to controller, Squirty | Evan, Crissy, Paul | June 1 |
| Hardware | Trim weight balance | Paul, Conway, Evan | 1 June |
| Hardware: Squirty | How often does it squirt? 2 x a day. We might want to go for 4 x a day if we can | Paul | May 22 |
| Hardware: Squirty | We have batteries. Ordered from PBS | Done | Done |
| Software: Triggering | Use 80% of max slope, from 1153 rolling mean | Rich, Crissy | Decided |
| Software: Collect schedule | Once it’s triggered, how long does it stay open for? Compare SES MARS flux estimates to Station M fluxes, make sure doesn’t clog. Aim to fill cup ¼ or less. Avoid clog danger zone. Based on Pulse 70 cups 6,9: < 30 mg C/ m^2/ d x 17 d collect time at Station M = 510 mg C/m^2. Estimating MARS flux could be as high as 1000 mg C/m^2/. So- let’s go for a 12 h collect time to be extra sure. | Crissy | ASAP |
| Software: Collect schedule | Once it has triggered, don’t add a wait period for the next trigger |  |  |
| Software: Collect schedule | Once it triggers, goes for how many cups? 1 cup, 12 hours each. | Crissy | ASAP |
| Software: Prep McLane trap | Configure trap schedule Configure interface with modem | Rich, Crissy, Paul | June 1 |
| SES prep | Configure trigger, Configure control triggering, Add weights and baffle, New batteries | Rich, Crissy, Paul | 1 June |
| Current meters | Change batteries, prep | Crissy | 15 May |
| How far from each other should/can they be deployed? | How far away would we deploy moorings from each other | Start with 1 km to make sure there’s enough time for transit. Deploy both then transit along the seafloor if they can go ½ a knot | Crissy and DJ Deploy McLane trap, Deploy SES, Dive on SES, transit to McLane trap and check that out. |
| Current meters- 2Dacmplus and stand | Stand | Only if easy | Paul to do, but only if easy |
| SES battery | 3 x an hour might get us 2.5 months | Paul | By next week |
| pH Sensor |  | Crissy |  |
| Deck test | Put both in test tank area on power, mission script And run tests of triggering TAKE OFF WIPER Add a piece of black paper on the slide to make sure it adds to data A brief in water test after bench tests | All | 15 May bench 1 June in water |
| Data analysis |  |  |  |
| Trigger algorithm simulation testing | Quantify false positives and negatives with different scenarios: SES data from MARS, SES data from Station M, Station M POC flux data | Crissy | After deployment |
| Modem | test |  | Get deck box and try to talk to it. |

Minutes 16 May 2024

Need cables between modem and controller- Paul is on it

Is there any benefit to connection serial port of mclane controller to modem so modem could record logs. Could this overcomplicate things? Rich is looking into it. Won’t mold cables until that’s figured out

Gonna hot dog the cable

Need to test the controller.

Mike weighed McLane trap. Evan put enough tabs on for two packs. Each float pack gives 25 pounds of lift. Might be a little off kilter but need it trimmed when sinking.