Autonomous detection and adaptive sampling of unpredictable carbon storage events monitored by a deep-sea optical sediment trap

Christine Huffard, Rich Henthorn, Irene Hu, Ben Jansen, Evan Mattiasen, Yanwu Zhang, Paul McGill, Colleen Durkin

500 words minimum, 2 pages including text, graphics and references

Quantifying, and eventually modelling, deep-ocean carbon storage requires detailed understanding of periods with unusually high activity in the biological carbon pump.  “Pulse” events, when large amounts of biological material sink to the deep-sea over a short period of time, are increasing in magnitude, duration, and frequency in the California Current (USA). They can account for a significant fraction of annual deep-ocean carbon storage in this region but remain unpredictable and hard to study.  This study presents progress toward targeted study of unpredictable pulse events through monitoring, event detection and command communication by a low-power instrument that then wakes up and initiates physical sample collection by a separate instrument with limited collection capacity.

Development began with a single deployment of the Sedimentation Event Sensor (SES), an optical sediment trap with high temporal sampling resolution, at the outer reaches of the Monterey Bay, California, USA (LAT LONG, depth). The SES imaged sinking material that accumulated on a slide at the base of its funnel following a 5-minute collection period, after which material was released back into the environment and the SES entered a 15-minute flow-through standby period. Images (5/13/2023—8/14/2023) were analyzed for attenuance, which is a unitless value that indicates the relative amount of material on the slide. This analysis provided a preliminary sense of scale, variability and patterns in biological carbon pump activity at this site, allowing us to define pulse events mathematically (at a scale amenable to testing of engineering developments), and develop a triggering algorithm that could indicate pulse event onset.

We added on-board image analysis and adaptive event-detection to the SES, and acoustic communication capabilities to a conventional sediment trap. After each sample collection the SES calculated attenuance and checked for rapid increases in marine snow accumulation. If a threshold was met, then the SES sent an acoustic “trigger” signal to initiate sampling by the nearby acoustics-enabled conventional sediment trap. This workflow aimed to detect the onset of, and collect during, pulse events. Sampling capacity of the conventional sediment trap was 21 cups, so overall goal was to develop algorithms that sent approximately that number of triggers.

In three field trials totaling over 12 months, the SES successfully detected unpredictable carbon storage events and sent acoustic commands (Table 1, Figure 1). To test communications, additional non-event triggers were sent on a scheduled basis, and if a certain period passed without event triggers. Those non-event triggers are not discussed here.  False positives (triggers when no event occurred) and negatives were evaluated….

Table 1. Deployment event triggers

| Start | End | Trigger algorithm | Event triggers (Rich please confirm which sent for these three columns) | Scheduled triggers |
| --- | --- | --- | --- | --- |
| 6/20/2024 | 9/18/2024 | Rapid increase and decrease trigger thresholds. Trigger anyway period = | 2024-07-01 10:38:00 2024-07-08 14:46:00 2024-09-04 16:02:00 2024-09-12 02:41:00 | 2024-06 24 0:05:00 2024-07-11 5:48:00 2024-08-22 16:37:00 2024-09-0416:32:00 |
| 12/4/2024 | 6/23/2025 | Toggle between two thresholds (high/low) based on trigger rate. Blackout period = Trigger anyway period = 21d | 2024-12-28 00:51:21 2025-01-26 03:49:44 2025-02-01 08:51:24 2025-02-02 22:50:36 2025-03-24 23:08:13 2025-04-08 22:34:12 2025-05-17 09:27:09 2025-06-05 10:02:43 | 2024-12-15 02:32:03 2025-02-13 02:22:11 2025-04-15 17:12:02 |
| 7/16/2025 | 11/10/2025 | Toggle between two thresholds (high/low) based on trigger rate. Blackout period = Trigger anyway period = 21d | 2025-08-23 04:51:14 2025-09-10 06:48:13 2025-09-11 10:18:14 2025-09-11 13:23:41 2025-09-11 23:33:34 2025-09-14 15:21:00 2025-09-18 09:41:20 2025-09-18 10:44:05 2025-09-18 11:47:29 2025-09-18 13:51:53 2025-09-18 14:53:10 2025-09-18 15:54:31 2025-09-18 16:58:08 2025-09-19 10:13:24 2025-09-28 11:30:39 2025-10-23 02:14:15 2025-10-23 15:40:09 2025-11-07 09:57:28 |  |

Plot atn for each: raw scatter blue alpha = 0.3, Rm150 as line, triggers as orange dot

Autonomous detection and adaptive sampling of unpredictable carbon storage events monitored by a deep-sea optical sediment trap

Christine Huffard, Rich Henthorn, Irene Hu, Ben Jansen, Evan Mattiasen, Yanwu Zhang, Paul McGill, Colleen Durkin

500 words minimum, 2 pages including text, graphics and references

Quantifying, and eventually modelling, deep-ocean carbon storage requires detailed understanding of periods with unusually high activity in the biological carbon pump.  “Pulse” events, when large amounts of biological material sink to the deep-sea over a short period of time, are increasing in magnitude, duration, and frequency in the California Current (USA). They can account for a significant fraction of annual deep-ocean carbon storage in this region but remain unpredictable and hard to study.  This study presents progress toward targeted study of unpredictable pulse events through monitoring, event detection and command communication by a low-power instrument that then wakes up and initiates physical sample collection by a separate instrument with limited collection capacity.

Development began with a single deployment of the Sedimentation Event Sensor (SES), an optical sediment trap with high temporal sampling resolution, at the outer reaches of the Monterey Bay, California, USA (LAT LONG, depth). The SES imaged sinking material that accumulated on a slide at the base of its funnel following a 5-minute collection period, after which material was released back into the environment and the SES entered a 15-minute flow-through standby period. Images (5/13/2023—8/14/2023) were analyzed for attenuance, which is a unitless value that indicates the relative amount of material on the slide. This analysis provided a preliminary sense of scale, variability and patterns in biological carbon pump activity at this site, allowing us to define pulse events mathematically (at a scale amenable to testing of engineering developments), and develop a triggering algorithm that could indicate pulse event onset.

We added on-board image analysis and adaptive event-detection to the SES, and acoustic communication capabilities to a conventional sediment trap. After each sample collection the SES calculated attenuance and checked for rapid increases in marine snow accumulation. If a threshold was met, then the SES sent an acoustic “trigger” signal to initiate sampling by the nearby acoustics-enabled conventional sediment trap. This workflow aimed to detect the onset of, and collect during, pulse events. Sampling capacity of the conventional sediment trap was 21 cups, so overall goal was to develop algorithms that sent approximately that number of triggers.

We present results from three field trials totaling over 12 months, in which the SES successfully detected unpredictable carbon storage events and sent acoustic commands (Table 1, Figure 1). To test communications, additional non-event triggers were sent on a scheduled basis, and if a certain period passed without event triggers. Those non-event triggers are not discussed here.  False positives (triggers when no event occurred) and negatives were evaluated based on timing of triggers in relation to pulse events (exceeding mean +2 σ).

Table 1. Deployment event triggers

| Start | End | Trigger algorithm | Event triggers | Scheduled triggers |
| --- | --- | --- | --- | --- |
| 6/20/2024 | 9/18/2024 | Rapid increase or decrease trigger thresholds. | 4 | 4 |
| 12/4/2024 | 6/23/2025 | Toggle between two thresholds (high/low) based on trigger rate. Trigger anyway period = 21d | 8 | 3 |
| 7/16/2025 | 11/10/2025 | Toggle between two thresholds (high/low) based on trigger rate. Trigger anyway period = 21d | 18 | 0 |

Crissy’s to-do list:

Recalc atn based on red channel code (overnight)

Recalc POC flux

Plot POC flux for each deployment: raw scatter blue alpha = 0.3, Rm150 as line, 		event triggers as orange dot

Use pulse definition to figure out how many false pos and negs per deployment:

- Value exceeds dataset mean + 2SD for at least 3 of the following datasets: raw values, 24H rolling mean, 3-day rolling mean, and 5-day rolling mean.

- “Pulse” starts with the first time this condition is met, and ends when the following 24 hours do not meet this condition.

Abstract Submission Guidelines

Submit an abstract summarizing:

- Problem statement

- Approach and methodology

- Key results (if available)

- Relevance to the OCEANS community

Abstracts are peer reviewed for acceptance.

- All abstracts must be submitted in PDF format and must be created by the author (co-authors are allowed).

- US Letter, English language

- 500-word minimum; 1000-word maximum; 2-page maximum – inclusive of text, graphics, and references.

- Please include at the top of the abstract the name of the principal author, the institute or affiliation the paper represents, and the title of the paper. We recommend using the IEEE conference template.

- Please note that the initial submission requires two items: a short abstract/summary (portal entry for indexing and initial processing, max 2250 characters) and an extended abstract (2-page PDF upload). Both must be submitted at the time of submission.

- The originally-submitted abstract cannot be used as a substitute for the conference paper. Please note that abstract PDFs do not need to be converted through the IEEE Xplore PDF eXpress service.

- Authors of accepted papers (regular and student poster competition) are expected to prepare and submit a conference paper (4 to 6 pages) for the conference proceedings (additional instructions will be provided at that time) and publish in IEEE Xplore®, subject to IEEE requirements.