| Project Number: | 902311 |
| --- | --- |

| Project Start: Jan. 1, | 2023 | Project Duration (in years): | 2 of 2-31 |
| --- | --- | --- | --- |

# Please indicate if proposal includes requests for any of the following (mark all that apply).

| Ship Time |  |  | ROV |  |  | AUV |  |  | Wave Glider |  |  | Mini ROV |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Please indicate what type of project is being proposed (mark only one).

|  | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Feasibility/Scoping [<100 days effort] |  |  | Development |  |  | Research |  |  | Infrastructure2 |  |  | Education |  |  | Time Series |  |  | Institute Initiative |  |  |

# Project Manager:		Brent Jones

Principal Investigator:		Andrew Hamilton

Lead Scientist:			Francisco Chavez

Lead Engineer:			Brent Jones

Project Title:		Power Generation onboard Wire Walker

# Please indicate which category best describes the project theme (mark only one).

|  |  |  |  |  |  |  |  |  |  | x |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ocean biogeochemistry |  |  | Ecosystem dynamics |  |  | Ocean visualization |  |  | Exploration & discovery |  |  |

Please indicate percentage of effort on this project anticipated for 2024 toward these three highest institutional goals from the Strategic Roadmap (percentage total should equal 100%).

| Goal | Innovate and Build | Explore and Protect | Inspire and Engage |
| --- | --- | --- | --- |
| Percentage | 75 | 0 | 25 |

- ABSTRACT (all projects)

Briefly provide a high-level summary of the project (ideally no more than 1/2 page).

Year 1:  Investigate several ideas for converting potential energy at depth.  These include (but are not limited to) turbines/pinwheels (continuation of previous project) and piston/leadscrew arrangements (unexplored but worth consideration).  Once a likely candidate is identified through bench studies a prototype will be manufactured and deployed on the Wire Walker. We anticipate much of this design can be implemented with printed parts, at least to the modest depths the system currently operates at MBARI.  The deployment will not impact the Wire Walker’s other project science requirements as we anticipate developing a self-contained unit that fits in existing Wire Walker payload space.  Deployment should take place at sea during a routine Wire Walker maintenance Paragon mission.

Year 2 and 3:  Further refine design based on year 1 knowledge gained.  Goals most likely center around energy generation efficiency and system reliability in the field.

- PROPOSAL (all projects)

# Background

Provide context for the project, being sure to address these questions:

- What fundamental problem does this project address, and why is it important?

- What makes this work relevant to MBARI and the Packard Foundation?

- Is this effort linked to other internal or externally funded projects or collaborative efforts?

- Where does this work fit into the ~5–10 year plan for your group, at MBARI, and in the field?

The WireWalker platform, developed at Scripps and marketed by Del Mar Oceanographic, has proven to be a successful vertical profiling platform for oceanographic measurements down to several thousand meters.  To date, hundreds of deployments have been made around the world. MBARI operates one of these units routinely used for science activities in the BOG group.  The vertical motion of this profiler is wave-powered by a ratcheting mechanism, but the instrumentation relies on batteries on the profiler, which naturally limits the instrumentation and cast frequency for extended deployments.  Wire-walker developer and Del Mar Oceanographic founder Rob Pinkle recently commented to us that “we mostly carry batteries up and down the wire”, and expressed that the instrumentation payload has been limited by energy needs.  The ability to further convert wave-energy to electrical energy on each cast would increase the amount and types of instrumentation that can be hosted on this profiler, and also increase the cast frequency possible during any given deployment.  The amount of electrical energy required per cast for the instrumentation is modest, and therefore it appears possible to generate enough energy on the down-cast to power the instruments on the up-stroke, enabling continuous cycling.

In 2009, MBARI had a scoping study project to develop an on-board electricity generation module for the Brooke Ocean “Seahorse” profiler, which operated on a similar principle to the Wire-walker, without the reliability the Del Mar team has achieved.  The concept explored then was to have the wave-powered profiler bring down a small evacuated volume on the down-cast, and then convert the resulting potential energy to electrical energy once at depth.  This of course requires the profiler to bring extra buoyancy down to depth, and tests in 2008 indicated this was not an impediment to the profiler’s operation.  The technique of integrating the energy capture during the entire down-cast and then converting quickly leads to some favorable power conversion efficiencies.  In 2009 this project was tabled due to reliability issues with the Brooke Ocean Technology profiler, and a funding opportunity to pursue larger-scale wave-energy conversion.

In the intervening time, SIO and Del Mar Oceanographic have perfected the wire-walker reliability, and MBARI has gained considerable experience with the implementation of the required power-electronics for this project.  The planned proposal now is to re-start this effort, and engage Del Mar Oceanographic and SIO as a partner for technology transfer should we be successful.  MBARI has an opportunity to develop and test the proposed device on-board the Wire-Walker we have as part of on-going science operations.  In this way we hope to increase our own science-data collection capabilities by demonstrating energy capture on-board this device, and contribute technology to the oceanographic community through a license agreement with Del Mar Oceanographic.  We have had an initial discussion with the Del Mar Oceanographic principles and they are interested.  They have had a long-term interest in this type of capability for their platform, and have done some prototyping of a different approach that involves a “wheel on the wire” design.  We expect that approach may be limited by efficiencies issues with the slow rotational rates that results, but we haven’t reviewed their design.  They view our suggestion as a development that is orthogonal to their approach and worth pursuing.  They have an expressed an interest in laying out the basics of a technology license agreement as early in this project as possible, so that everyone knows what to expect.  This is in contrast to an approach in which we develop the technology on speculation and pursued a license agreement later.  Should we proceed, we would aim to get such a negotiation to the level a of term-sheet draft by the proposal due date so that a rational decision can be made about the value of this technology transfer path.

In our discussions with Del Mar Oceanographic, it also became clear that they struggle with communications from the wire-walker, and are limited by the speeds of the inductive links they employ.  The Niobicon AUV docking technology also may have a direct application to this platform, for data transfer during at least part of the cast, or power transfer from a solar source on the buoy.  It is not clear at the moment how such a technology transfer to this platform would work with the Northrop agreement, but such an approach could preclude the need for this project and that path will be examined as we prepare this proposal.

Nature and Scope of the Proposed Project

Please provide a sense of the scale and scope of the proposed work. Is this a large de novo R&D effort, modest platform/sensor integration, targeted science investigation with minimal or no engineering needs, extension of current program, small-scale scoping exercise expected to lead to a larger effort, or other? If the project is intended to be multiple years, how will the anticipated level of effort likely change over time?

Year 1:  Investigate several ideas for converting potential energy at depth.  These include (but are not limited to) turbines/pinwheels (continuation of previous project) and piston/leadscrew arrangements (unexplored but worth consideration).  Once a likely candidate is identified through bench studies a prototype will be manufactured and deployed on the Wire Walker. We anticipate much of this design can be implemented with printed parts, at least to the modest depths the system currently operates at MBARI.  The deployment will not impact the Wire Walker’s other project science requirements as we anticipate developing a self-contained unit that fits in existing Wire Walker payload space.  Deployment should take place at sea during a routine Wire Walker maintenance Paragon mission.

Year 2 and 3:  Further refine design based on year 1 knowledge gained.  Goals most likely center around energy generation efficiency and system reliability in the field.  Labor and shared resource requests should increase compared to year 1.

Does this activity relate to one or more 2024 proposals your team is submitting?

If so, provide a list of relevant projects by project/DQ # and provide a brief description of how they are related. Since resources are limited and cuts during Phase 1 are likely, please indicate where this project fits in the hierarchy of your priorities.  Does this proposal take precedent over other ongoing or planned activities?

Yes, this project would be part of a family of technology development projects that have the goal of enabling and extending a persistent presence of autonomous oceanographic measurements through energy capture and transfer technology development. In particular, related projects are:

- 901405:  Wave-Energy for Oceanographic Instrumentation

- 708167: WEC-Dynamics and Control (Sandia National Labs)

- 705102: Open Wave-Energy Control System Research Platform (DOE)

- 902106: Niobicon AUV dock dev

Although this project shares the theme of energy harvesting with many of these projects, the strongest overlap is with the 901405 which would provide the power-electronics basis for this project. As a new effort, this project would necessarily need to be ranked as the last priority in this collection of efforts, unfortunately.

Anticipated Shared Resource Needs

What staffing is required to advance the work in 2024 (e.g., is this a multi-faceted engineering development or a targeted science investigation that does not require significant use of shared personnel)? Is in-house or contract manufacturing needed? What level of ship/ROV/AUV (DMO) support is likely? If this is intended to be a multi-year effort, please provide a rough indication of anticipated shared resource needs over the duration of the project (e.g., will stay flat, will significantly increase in Year X, will significantly decrease by Year Y).

This effort will require approximately 2.5 months of support from Jones, Jensen, Marin, Stern, and Hamilton.  Additionally, several Paragon days and a few weeks of machinist time will be requested.  Shared resource needs are expected to stay flat in year two and three.

Anticipated Expenses

What rough order of magnitude direct expense budget is anticipated annually over the course of the project? Is procurement of specialized capital equipment or outside contractors required? Will there be annual travel?

<$2k per year and no specialized capital equipment, outside contractors, nor travel are anticipated.

Intellectual Property (IP) Considerations

Do you anticipate a need for a cooperative research and development agreement or licensing arrangement with an external entity? Will the work potentially result in a patentable product or service? Will novel IP and/or custom-made equipment be created, and how might it be disseminated (e.g., license agreement and/or collaborations)?

For additional guidance please review MBARI’s intellectual property internal web page: https://mww.mbari.org/itd/ip/.

Yes, ideally a licensing agreement with Del Mar Oceanographic would be negotiated quite early in this project, making Del Mar a partner with us in this development, and more importantly providing a technology transfer path if we are successful.  Unfortunately, due to circumstances beyond our control, we were not able to get anything in place before submitting this proposal.  We will work towards a NDA with Del Mar Oceanographic this year in anticipation of a licensing agreement put in place during 2023 (will be a stated deliverable).

Data

What data will be generated? How will they be managed/stored? Are any special data visualization needs anticipated?

Data from this project will be limited to engineering test data of modest size stored locally on participant laptops/desktops.  No data visualization is needed

Anticipated Impact

Who are the consumers of the instruments, methods, technology, and data that will be generated within and beyond MBARI? Will the project create tangible connections with other organizations (e.g., academic, NGO, MBA, MBNMS, state or federal agency)?

None anticipated.

Anticipated Outreach Needs

Thinking about the audiences/consumers you highlighted in the question above, do you anticipate needing to create outreach assets to help increase your impact? If so, please specify whether you anticipate needing to work with ITD and also specify what type of asset (story, graphic, animation, video, etc.) you might need.

None anticipated.

Continuing Projects Only: Status Report

Briefly provide an update of the current project relative to what was proposed for 2023 by responding to the prompts below. If the project includes multiple subproject projects, please list each of these under a subheading as you deem appropriate.

# Current status of project and results to date

[Click here to enter text]

# What percentage of labor and budget have been used through the 1st half of year?

We have used 24% labor and 1% of expenses

Will you complete the current year’s scheduled work?

The primary goals for 2023 are to evaluate the Wire Walker platform performance with 1L additional buoyancy, identify a mechanical technology for energy harvesting, and produce a prototype instrument for use on Wire Walker.

Tests on wire walker in the beginning of 2023 demonstrate it can carry a variable 0-1L payload with no significant performance degradation.

We anticipate selecting a harvesting technology by EOY 2023.  We have empirical results for a turbine wheel and simulated results for a piston system.  A suitable bench test piston has been sourced (re-tasking an old SOLO I float).  Bench testing on this system during the second half of 2023 should produce results that will allow us to decide between the turbine or piston.

We don’t anticipate a complete prototype ready for installation on Wire Walker in 2023.  However, we hope to have a viable bench test mechanical prototype capable of deployment.

# Significant changes from previous project proposal

A fully functioning mechanical and electrical prototype deployment will now take place by end of year 2024.  We would like to increase the number of hours requested for Brent Jones and George Stern for design, fabrication, and coding of a microcontroller energy harvesting electrical hardware system.

Over the past two years, what peer-reviewed publications, education and/or outreach activities have resulted directly from this project? Please be brief.

none

All Projects: Specific Plans for 2024

# Research Activities

None

# Engineering/technology development

Completion of a prototype mechanical + electrical system deployed on a Wire Walker

# Field programs (detail ship requirements/deployments/diver requirements)

# In addition to describing anticipated field programs, please be sure to include requirements for specific platforms and equipment (Dorado AUVs, LRAUVs, Wave Glider, mini ROV, etc.) for each proposed field program so that expectations are clear. Requests must also be entered in the budget entry system.

Paragon (6 days).  We’d like to combine Paragon ops with scheduled Wire Walker maintenance runs under the CANON project.  Brent Jones will be captain.

# Specific deliverables

- Complete a viable prototype system for energy harvesting that includes:

  - Provides an energy source/storage that extends Wire Walker deployment duration in an impactful way

  - Can be physically integrated on Wire Walker with no negative impact to existing vertical travel characteristics

  - Does not impact existing sensors

- Demonstrate system on the lab bench and in the field

# Milestones and required personnel (see personnel classification list for staff classifications)

Andrew Hamilton (40 hrs)

Brent Jones (160 hrs)

Scott Jensen (40 hrs)

Roman Marin (40 hrs)

George Stern (120 hrs)

| 2024 Project Start Date: MM/DD/2024 | Milestone 1 | Milestone 2 | Milestone 3 | Milestone 4 | Milestone 5 | Total Resources (in hours) for each resource category for all milestones listed here |
| --- | --- | --- | --- | --- | --- | --- |
| Milestone Date | 6/2024 | 12/2024 |  |  |  |  |
| Milestone Description | Working bench prototype | Working prototype deployed |  |  |  |  |
| Electrical Engineer | 200 | 120 |  |  |  | 320 |
| Software Eng-Embedded |  |  |  |  |  |  |
| Software Eng-Info Eng |  |  |  |  |  |  |
| Mechanical Engineer | 20 | 20 |  |  |  | 40 |
| Mechanical Eng-Division |  |  |  |  |  |  |
| Mechanical Eng-Mfg |  |  |  |  |  |  |
| Associate Engineer |  |  |  |  |  |  |
| Electrical Tech |  |  |  |  |  |  |
| Software Tech |  |  |  |  |  |  |
| Mechanical Tech | 40 |  |  |  |  | 40 |
| Machinist |  |  |  |  |  |  |
| Divers |  |  |  |  |  |  |
| Video Lab |  |  |  |  |  |  |
| (Add other categories/rows as needed) |  |  |  |  |  |  |

# Budget justification

$2k misc mechanical and electrical parts for prototype.

# Project team

Andrew Hamilton (40 hrs) –mechanical design direction

Brent Jones (160 hrs) – Project management, prototyping, bench testing, field deployment

Scott Jensen (40 hrs) – electrical eng support for prototype

George Stern (120 hrs) – electrical eng support for prototype

Roman Marin (40 hrs) – mechanical support for prototype

# External consultants/contractors

If you propose to engage an external consultant/contractor, please attach a sufficiently detailed SOW and budget proposal from the vendor. If approved, a fully executed Independent Contractor Agreement must be in place before work can begin.

Not anticipated

# Project lab and office space requirements

In addition to any changes to current space usage, please also include anticipated office and lab needs for students, postdocs, collaborators, etc.

No changes to current space usage

# Risk matrix (development and infrastructure projects only)

|  | Technical Risk | Organizational Risk | Tech Transfer Risk | Labor Estimate Risk |
| --- | --- | --- | --- | --- |
| High |  |  |  |  |
| Medium | x |  |  |  |
| Low |  | x | x | x |

Compliance Considerations

All responses must be recorded in the Google form linked below. A form must be submitted for every project.

Link to Google Compliance Considerations form -OR- https://forms.gle/wVCb9VRRuYBU5yvF8

| # CATEGORY | # Yes/No | # DETAILS |
| --- | --- | --- |
| # Isotope Use (see policy and cruise planning) Will the project use stable or radioisotopes? | No |  |
| # Battery waste over 4 liters (1 gallon) Indicate type and amount. | No |  |
| # Liquid hazardous waste over 60 liters (15 gallons) # Indicate composition and quantity. | No |  |
| # Export Regulation Foreign collaborations or international travel? | No |  |
| # Technology access # Any potential exposure to proprietary information about export controlled equipment? | No |  |
| # Export controlled purchases # Items purchased/in possession or to be purchased that are not on list? | No |  |
| # Export controlled items (hardware, software, tech data) # Items to be transferred outside US (temporarily or permanently)? To what countries (including International Waters)? | No |  |
| # Encryption Items # Use of non-commercial (specially licensed, non-mass-market) encryption or information security software? | No |  |
| # Work covered by Marine Sanctuary permit? Yes, if work falls under blanket authorization. | Yes |  |
| # Work conducted in SESAs? # ID # for Sanctuary Ecologically Significant Areas | No |  |
| # Equipment deployed with anchors? # Give details for deployment; recovery if < 500lbs | No | Piggy backing on existing Wire Walker under CANON |
| # CA Fish & Wildlife permit required for samples? | No |  |
| # Working in State Marine Protected Area? | No |  |
| # Require Coast Guard PATON for buoys? # Private Aid to Navigation. Permit in hand? | No |  |
| # Require Local Notice to Mariners Indicate who will be submitting | No |  |
| # Permits from other agencies? | No |  |
| # Use of active acoustics not already on list? | No |  |

This date and number should remain the same for the duration of the project. If your project start date for 2024 is not January 1, please indicate the start date in the first cell of the milestone chart. Format for duration example: “Year 2 of 5”.

Development and infrastructure projects must complete the risk matrix (see end of the proposal form).

Not all projects will require all four titles to be filled in. Please indicate individuals only for those titles that are appropriate for your project.

This category includes transfer of hardware, know-how, publications, presentations, etc.

This number should be the same number you entered on the web budget resource requests for this particular resource.