Pre-Deployment Checklist

Version 2Jun26

For clock sync:

ssh: sintela@192.168.1.236

user: sintela

password: 5intela_440

Instrument set-up

- Check Engineering set-up (decimator, leaky cutoff frequency settings). If changes are made need to click save chain in the top right of the page.

- Check that correct optical set-up configuration is applied and automatically applied on reboot.

- Check noise floor is reasonable.

- Run OTDR to make sure there is no significant energy loss along cable or reflection at termination. Make sure data recorders are off before running.

- Check that GPS time setting is correctly enabled/disabled.

- Check that there is sufficient memory storage available: _______ MB available.

- Check that data recorders are set-up and files are being recorded to the right place.

- Check RAID is active (especially after deleting files, may need to configure again).

Instrument test via deck cable

- Install shorting plug.

- Apply power to deck cable with 30 V bench power supply.

- Confirm communication with DAS (ping, login to Sintela dashboard).

- Remove deck cable and plug in DeepSea Connect can.

Instrument test via DeepSea Connect (DSC)

- Remove zipties binding instrument DSC can.

- Ask ROV pilots to power ROV side of DSC.

- Test communication to ROV side of DSC by pinging address ?.?.?.?.

- Engage magnet to turn instrument DSC can ON.

- Test communication with instrument DSC can by pinging address ?.?.?.?.

- Test communication with DAS by pinging address ?.?.?.? (or SSH to command prompt?).

- Power OFF DSC can by pushing ROV handle in.

MOB Day

- Test instrument via deck cable (see above).

- Test instrument via DeepSea Connect (see above).

- Test DAS recorders.

Pre-Dive

- Test instrument via deck cable and check set-up (see above).

- Synchronize time.

- Confirm shorting plug has o-rings and is in place.

- Connect batteries after applying fresh grease to connectors.

- Verify working, start data recorders: recording started at ________PDT

- Turn off power supply. Verify working on battery power.

- Close DAS application.

- Disconnect and stow deck cable and connect DSC after applying fresh grease to connector.

- Test instrument via DeepSea Connect.

- Test DSC ROV side working.

- Make sure data recorders are on.

- Confirm all connectors are snug and all cables are tied up.

- Confirm DSC ROV handle is in OFF position.

- Confirm deck cable is removed and DSC can is plugged in.

- Check comp levels: levels are __________________________

- Record beacon serial numbers and RF frequencies.

- Turn ON beacons (e.g. RF beacon and strobe) and pull any safety pins (e.g., drop weights).

- Notify deck crew to deploy instrument.