Geosense Recovery

Recovery Steps for 6/10/2026:

- Ping RT6 release: 2013, TAT: 240, UID: 0066F4, S/N: 312700-001

- ROV dive at location of spool: 3647.3238N, 12153.60W.

- Detect elevator with a homer beacon.

- Do not plant to connect using Deep Sea Connect.

- Position ship, trigger Acoustic Release.

  - Release elevator. RT6 release, RF beacon (frequency = 156.67500 Mhz),

- Recover Elevator and secure on deck.

- Fiber will be hanging down from the ship.

  - 115 m fiber in ~400 m of water

  - Position vessel such that current pushes cable away from the vessel.

  - Maneuver the ship so not to damage the fiber by the props.

- When Elevator on deck, disconnect the battery from DAS sphere.

- Recover the cable, 2-3 people, garbage can; try to coil over-under.

  - Adjustable zip ties or Velcro.

  - Garbage can.

  - Something to smooth out the fiber over the edge/ increase radius?

- Stash the fiber into the Crate on Elevator.

On-Deck Steps:

- Make sure the Elevator is secured on the deck, and the system is secure on the elevator.

- Make sure the Fiber is secured on the elevator frame/ Crate on Elevator.

- If fiber is severed, secure the end of the fiber so the laser radiation is contained and not radiated on the deck.

- Cover the fiber loop and fiber spool with black felt, set area Laser Warning sign and Unattended Laser Operation signs(s).

- Disconnect the Deep Sea Connect cable from the DAS instrument sphere; install dummy plug on the Deep Sea Connect cable.

- Prepare Deck Cable; run deck cable from the science van to the elevator.

- Connect Deck Cable banana plugs to the multimeter to measure voltage of the DAS instrument battery spheres.

- Connect the Deck Cable RG45 connector to the notebook.

- Plug deck cable to the DAS instrument sphere connector.

- Measure the voltage back-feed at the Deck Cable banana plugs.

- Plug Deck Cable banana plugs into Extech power supply, apply 28.5V.

- Try to talk to the instrument with the notebook and measure RTC clock drift.

- Check the time stamp of the last recorder data file.

Onshore Steps:

- Rinse off equipment.

- Download Geosense data files.

- Download hydrophone data.

- Visually inspect cable.

- Run OTDR (internal and Bryan’s device) and check noise floor.

- Straighten cable, run OTDR and check noise floor.

Items to bring/load:

- Palletainer or garbage can (for the fiber).

- Adjustable zip ties or Velcro

- Interrogator interface devices (dummy plugs, deck cable, power supply, laptop)

- Laser containment supplies (black felt, goggles)

Safety:

- Short laser safety orientation for the R/V Rachel Carson crew to address a scenario that the system is still working, and optical fiber is severed during the recovery.

- Crew members on the port side near elevator recovery area, will need to wear LG11/ LG11a laser safety goggles during the recovery, up until the system is secured on the deck and system power removed / battery sphere disconnected from DAS sphere.

- Make sure individual fit with goggles LG11 and LG11a which are the same optically but have different frames.

- Key points:

  - “If you see the source of the hazard, then you are protected”

  - “Make sure google sit firmly on, with no gaps around”