Geosense Recovery

Recovery Steps:

- Ping RT6 release (address: 2013, TAT: 240, UID: 0066F4, S/N: 312700-001)

- ROV dive at location of spool: 3647.3238N, 12153.60W.

- Pick up spool and bring it back to elevator.

- Put spool down and survey around elevator – to capture cable position and the location of any cable strumming.

- Secure spool on elevator.

- Lift and lower/putdown elevator with ROV to remove sediment from elevator base.

- Recover ROV.

- Release elevator. RT6 release,  RF beacon (frequency = 156.67500 Mhz),

- Recover elevator and secure on deck.

- Position vessel such that current pushes cable away from the vessel.

- Place both cable ends through a block to bring them off the deck and also elevate the cable out of the water near the ship.

- Recover cable by pulling or reeling in both ends on deck.

- Before reaching the end, remove cable from block.

On-Deck Steps:

- Make sure the Elevator is secured on the deck, and the system is secure on the Elevator.

- Make sure the fiber is secured on the elevator frame.

- If fiber is severed, secure the end of the fiber so the laser radiation is contained and not radiated on the deck.

- Cover the fiber loop and fiber spool with black felt, set area Laser Warning sign and Unattended Laser Operation signs(s).

- Disconnect the Deep Sea Connect cable from the DAS instrument sphere; install dummy plug on the Deep Sea Connect cable.

- Prepare Deck Cable; run deck cable from the science van to the elevator.

- Connect Deck Cable banana plugs to the multimeter to measure voltage of the DAS instrument battery spheres.

- Connect the Deck Cable RG45 connector to the notebook.

- Plug deck cable to the DAS instrument sphere connector.

- Measure the voltage back-feed at the Deck Cable banana plugs.

- Plug Deck Cable banana plugs into Extech power supply, apply 28.5V.

- Try to talk to the instrument with the notebook and measure RTC clock drift.

- Check the time stamp of the last recorder data file.

Onshore Steps:

- Rinse off equipment.

- Download Geosense data files.

- Download hydrophone data.

- Visually inspect cable.

- Run OTDR (internal and Bryan’s device) and check noise floor.

- Straighten cable, run OTDR and check noise floor.

Items to bring/load:

- Palletainer

- Interrogator interface devices (dummy plugs, deck cable, power supply, laptop)

- Laser containment supplies (black felt, goggles)