Notes from Meeting with Sintela Feb 14, 2024 – these are Alana’s notes

- MBARI to confirm temp at the site for Sintela

- Ideal for Sintela is instrument to be sitting at 25 deg

- Freq range is 0-10 Hz,  up to 20 Hz (not sure if this is a note saying what we want or a spec?)

- Instrument is orientation dependent (I believe this impacts temp control)

- Works best if housing is purged with dry air, not nitrogen

- We should be able to sense al of the 1 km length

- A note about optisense noise??

- Standard memory on unit can hold 8 TB of data

- Some room to configure the Orin to optimize power

- Can we provide data from other DAS as an example of amplitude of signals for our instrument

- Second disk to provide redundancy

- Birns connector will probably be fine but the APR is better

- Default clock is 1ppm, drift of 3 sec per month

- We will be able to set fixed ip for instrument

- Sintela voltage range 16-78V our +28V battery pack will work in their range

- We could possibly rent onyx to run on MARS simultaneously

- Yes there is an API and full access to embedded algorithms, running nvdia orin nano with 8 GB RAM

- Yes there are event hooks, there are highly configurable signal processing building blocks

- We can get docker image

- Sample data to look at is available