Pre-Deployment Checklist

For clock sync:

ssh: sintela@192.168.1.236

user: sintela

password: 5intela_440

Instrument set-up

- Check Engineering set-up (decimator, leakycutoff frequency settings). If changes are made need to click save chain in the top right of the page.

- Check that correct optical set-up configuration is applied and automatically applied on reboot.

- Check noise floor is reasonable.

- Run OTDR to make sure there is no significant energy loss along cable or reflection at termination. Make sure recorders are off before running.

- Check that GPS time setting is correctly enabled/disabled.

- Check that there is sufficient memory storage available.

- Check that recorders are set-up and files are being recorded to the right place.

- Check RAID is active (especially after deleting files, may need to configure again).

Instrument test via deck cable

- Install shortening plug.

- Start power.

- Confirm communication with DAS (ping, login to Sintela dashboard).

- Remove deck cable and plug in DeepSea Connect can.

Instrument test via DeepSea Connect (DSC)

- Remove zipties binding DSC can.

- Test communication to “ROV”.

- Engage magnet to turn DSC can on.

- Test communication with DSC can.

- Test communication with DAS.

- Disable DSC can by pushing ROV handle in.

MOB Day

- Test instrument via deck cable (see above).

- Test instrument via DeepSea Connect (see above).

- Test recorders.

Pre-Dive

- Test instrument via deck cable and check set-up (see above).

- Synchronize time.

- Confirm shortening plug has o-rings and is in place.

- Connect batteries.

- Verify working, start recorders.

- Turn off power supply. Verify working.

- Close application.

- Disconnect deck cable.

- Test instrument via DeepSea Connect.

- Test DSC ROV side working.

- Make sure recorders are on.

- Confirm all connectors are snug and all cables are tied up.

- Confirm DSC ROV handle is in off position.

- Confirm deck cable is removed and DSC can is plugged in.

- Check comp levels.

- Turn on sensors (e.g. RF beacon and strobe) and pull any safety pins (e.g., drop weights)